unit buzDatum;

{ GeographicProjection.cpp

Projection.cpp

DatumConvertor.cpp}

interface

{uses buzGlobal; }

{uses SysUtils,Dialogs,Math;}

Procedure ConvertDatums( {dXTemp,dYTemp,DXdtemp,DYdtemp:Double);}

LongXCoord, LatYCoord:Double;

var XCoord, YCoord:Double);

Function convertFromWGS84(var lat, lon, height:Double{ DATUM to }):Boolean;

Function convertToWGS84(var  lat, lon, height:Double{ DATUM from }):Boolean;

Function getEllipsoidParameters( ELLIPSOIDe:String ):Boolean;

Function getDatumEllipsoid( DATUMd:Integer ):String;

Function getDatumParameters( DATUMd:Integer ):Boolean;

Procedure FillDatumEllipsoids;

{  static bool convertDatum( double& lat, double& lon, DATUM from, DATUM to );

static ELLIPSOID getDatumEllipsoid( DATUM d );

static bool convertToWGS84( double& lat, double& lon, double& height,

DATUM from );

static bool convertFromWGS84( double& lat, double& lon, double& height,

DATUM to );

static DatumParameters* getDatumParameters( DATUM d );

static EllipsoidParameters* getEllipsoidParameters( ELLIPSOID e );

/*static bool convertToECEF( double& lat, double& lon, double& height,

DATUM from );

static bool convertFromECEF( double& lat, double& lon, double& height,

DATUM to );*/

// Conversion parameters array

static DatumParameters     d_datumParams[];

static EllipsoidParameters d_ellipsoidParams[];

}

implementation

uses buzGlobal;

TYPE

DatumParameters=Record

DATUMd_datum,

ELLIPSOIDd_ellipsoid:String;

d_deltaX,

d_deltaY,

d_deltaZ:double;

end;

// Ellipsoid info type

EllipsoidParameters=Record

ELLIPSOIDd_ellipsoid:String;

d_deltaA, // equatorial radius shift from WGS84

d_deltaF:double; // flattening shift from WGS84

end;

var

WorkDatum :DatumParameters;

WorkEllipsoid:EllipsoidParameters;

// Define WGS84 constants

{#define PI 3.1415926535 }

WGS84_RADIUS:double= 6378137.0;

WGS84_FLATTENING:double= ( 1.0 / 298.257223563 );

// Initialize datum conversion parameters array

//DatumConvertor::DatumParameters DatumConvertor::

d_datumParams: array[0..23] of DatumParameters;

d_ellipsoidParams: array[0..13] of EllipsoidParameters;

{Already Radians}

Procedure ConvertDatums( {dXTemp,dYTemp,DXdtemp,DYdtemp:Double);}

LongXCoord, LatYCoord:Double;

var XCoord, YCoord:Double);

var  lat, lon, height:double;

Begin

// XXX - Should I be getting the actual height for this location from a

// table or something?  The height doesn't seem to effect the results much,

// at least within the accuracy limits of USGS data.

height := 0.0;

lat:=LatYCoord;

lon:=LongXCoord;

// Convert the point to WGS84

if ( inparm[18]<>23 {WGS_84 != from} )then

begin

if (not convertToWGS84(lat, lon, height{, from} ) )then

begin

DoProjError(2002,2002,1,1 ,inparm[18]);

Exit;

end;

end;

// Convert the point from WGS84

if (outparm[18]<>23{ WGS_84 != to} )then

begin

if (not convertFromWGS84( lat, lon, height{, to} ) )then

begin

DoProjError(2002,2002,2,1 ,inparm[18]);

Exit;

end;

end;

YCoord:=lat;

XCoord:=lon;

End;

Function convertFromWGS84(var  lat, lon, height:Double):Boolean;

var

z,deltaA,deltaF,fromA,fromF,fromES,bda,sLat,cLat,sLon,

cLon,dx,dy,dz,tmp,sqrtTmp,rn,rm,dLat,dLon,dHeight

:Double;

Begin

{ DATUM to

DatumParameters=Record

DATUMd_datum,

ELLIPSOIDd_ellipsoid:String;

d_deltaX,

d_deltaY,

d_deltaZ:double;}

If (not getDatumParameters(Round(inparm[18])))

then

Begin

DoProjError(2002,2002,3,1 ,inparm[18]);

Result:=False;

Exit;

End;

z := height;

// Get the ellipsoid parameters

{ EllipsoidParameters=Record

ELLIPSOIDd_ellipsoid:String;

d_deltaA, // equatorial radius shift from WGS84

d_deltaF:double; // flattening shift from WGS84}

If (not getEllipsoidParameters( WorkDatum.ELLIPSOIDd_ellipsoid ))

then

Begin

DoProjError(2002,2002,4,1 ,inparm[18]);

Result:=False;

Exit;

End;

// Initialize parameters

deltaA := -WorkEllipsoid.d_deltaA;

deltaF := -WorkEllipsoid.d_deltaF;

fromA := WGS84_RADIUS;

fromF := WGS84_FLATTENING;

fromES := ( 2.0 - fromF ) * fromF;

bda := 1.0 - fromF;

sLat := sin( lat );

cLat := cos( lat );

sLon := sin( lon );

cLon := cos( lon );

dx := -WorkDatum.d_deltaX;

dy := -WorkDatum.d_deltaY;

dz := -WorkDatum.d_deltaZ;

// Calculate the radius of curvature in prime veritcal

tmp := 1.0 - fromES * sLat * sLat;

sqrtTmp := sqrt( tmp );

rn := fromA / sqrtTmp;

// Calculate the radius of curvature in prime meridian

rm := fromA * ( 1.0 - fromES ) / ( tmp * sqrtTmp );

// Calculate the difference in latitude

dLat := ( -dx * sLat * cLon - dy * sLat * sLon ) + dz * cLat;

dLat := dLat +(( deltaA * rn * fromES * sLat * cLat ) / fromA);

dLat := dLat +(deltaF * ( rm / bda + rn * bda ) * sLat * cLat);

dLat := dLat /(rm + z);

// Calculate the difference in longitude

dLon := ( -dx * sLon + dy * cLon ) / ( cLat * ( rn + z ) );

// Calculate the difference in height

dHeight := dx * cLat * cLon + dy * cLat * sLon + dz * sLat -

( deltaA * fromA ) / rn;

dHeight := dHeight+(deltaF * bda * rn * sLat * sLat);

lat := lat+dLat;

lon := lon+dLon;

height := height+dHeight;

Result:= true;

End;

Function convertToWGS84(var  lat, lon, height:Double):Boolean;

var

z,deltaA,deltaF,fromA,fromF,fromES,bda,sLat,cLat,sLon,

cLon,dx,dy,dz,tmp,sqrtTmp,rn,rm,dLat,dLon,dHeight

:Double;

Begin{ DATUM from }

// Use z as zero for now

z := height;

// Get the datum parameters

If (not getDatumParameters(Round(outparm[18])))

then

Begin

DoProjError(2002,2002,5,1 ,inparm[18]);

Result:=False;

Exit;

End;

// Get the ellipsoid parameters

If (not getEllipsoidParameters( WorkDatum.ELLIPSOIDd_ellipsoid ))

then

Begin

DoProjError(2002,2002,6,1 ,inparm[18]);

Result:=False;

Exit;

End;

// Initialize parameters

deltaA := WorkEllipsoid.d_deltaA;

deltaF := WorkEllipsoid.d_deltaF;

fromA := WGS84_RADIUS - deltaA;

fromF := WGS84_FLATTENING - deltaF;

fromES := ( 2.0 - fromF ) * fromF;

bda := 1.0 - fromF;

sLat := sin( lat );

cLat := cos( lat );

sLon := sin( lon );

cLon := cos( lon );

dx := WorkDatum.d_deltaX;

dy := WorkDatum.d_deltaY;

dz := WorkDatum.d_deltaZ;

// Calculate the radius of curvature in prime veritcal

tmp := 1.0 - fromES * sLat * sLat;

sqrtTmp := sqrt( tmp );

rn := fromA / sqrtTmp;

// Calculate the radius of curvature in prime meridian

rm := fromA * ( 1.0 - fromES ) / ( tmp * sqrtTmp );

// Calculate the difference in latitude

dLat := ( -dx * sLat * cLon - dy * sLat * sLon ) + dz * cLat;

dLat := dLat+(( deltaA * rn * fromES * sLat * cLat ) / fromA);

dLat := dLat+(deltaF * ( rm / bda + rn * bda ) * sLat * cLat);

dLat := dLat/(rm + z);

// Calculate the difference in longitude

dLon := ( -dx * sLon + dy * cLon ) / ( cLat * ( rn + z ) );

// Calculate the difference in height

dHeight := dx * cLat * cLon + dy * cLat * sLon + dz * sLat -

( deltaA * fromA ) / rn;

dHeight :=dHeight + deltaF * bda * rn * sLat * sLat;

lat := lat+dLat;

lon := lon+dLon;

height := height+dHeight;

Result:= true;

End;

(*inline bool

Projection::checkCoordinateRange( double latitude, double longitude ) const

{

return ( ( latitude <= 90.0 && latitude >= -90.0 ) &&

( longitude <= 180.0 && longitude >= -180.0 ) );

}

*)

(*

/*

bool

DatumConvertor::convertToECEF( double& lat, double& lon, double& height,

DATUM from )

{

// Get the conversion parameters

DatumParameters* pParams = getConversionParameters( from );

if ( NULL == pParams )

{

return false;

}

// Initialize parameters

double deltaA = pParams->d_deltaA;

double deltaF = pParams->d_deltaF;

double fromA = WGS84_RADIUS - deltaA;

double fromF = WGS84_FLATTENING - deltaF;

double fromES = ( 2.0 - fromF ) * fromF;

double polarRadius = fromA * ( 1.0 - fromF );

double sLat = sin( lat );

double cLat = cos( lat );

double sLon = sin( lon );

double cLon = cos( lon );

double N = fromA / sqrt( 1 - fromES * sLat * sLat );

lon = ( N + height ) * cLat * cLon + pParams->d_deltaX;

lat = ( N + height ) * cLat * sLon + pParams->d_deltaY;

height = ( N * ( 1.0 - fromES ) + height ) * sLat + pParams->d_deltaZ;

return true;

}

bool

DatumConvertor::convertFromECEF( double& lat, double& lon, double& height,

DATUM to )

{

// Get the conversion parameters

DatumParameters* pParams = getConversionParameters( to );

if ( NULL == pParams )

{

return false;

}

lon -= pParams->d_deltaX;

lat -= pParams->d_deltaY;

height -= pParams->d_deltaZ;

// Initialize parameters

double deltaA = pParams->d_deltaA;

double deltaF = pParams->d_deltaF;

double toA = WGS84_RADIUS - deltaA;

double toF = WGS84_FLATTENING - deltaF;

double toES = ( 2.0 - toF ) * toF;

double polarRadius = toA * ( 1.0 - toF );

double p = sqrt( lat * lat + lon * lon );

double theta = atan( height / p );

double e2s = ( toA * toA - polarRadius * polarRadius )

/ ( polarRadius * polarRadius );

double Y = lat;

lat = atan( ( height + e2s * polarRadius * sin( theta ) * sin( theta )

* sin( theta ) ) / ( p - toES * toA * cos( theta ) * cos( theta )

* cos( theta ) ) );

lon = atan2( Y, lon );

double sLat = sin( lat );

double N = toA / sqrt( 1 - toES * sLat * sLat );

height = p / cos( lat ) - N;

return true;

}*)

Function  getEllipsoidParameters( ELLIPSOIDe:String ):Boolean;

{d_ellipsoidParams: array[0..13] of EllipsoidParameters;}

var I:Integer;

Begin

Result:=False;

For I:=0 to 13 do

Begin

If (ELLIPSOIDe =d_ellipsoidParams[I].ELLIPSOIDd_ellipsoid ) then

begin

WorkEllipsoid.ELLIPSOIDd_ellipsoid:=

d_ellipsoidParams[I].ELLIPSOIDd_ellipsoid;

WorkEllipsoid.d_deltaA:=

d_ellipsoidParams[I].d_deltaA;

WorkEllipsoid.d_deltaF:=

d_ellipsoidParams[I].d_deltaF;

Result:=True;

end;

End;

End;

{ELLIPSOID DatumConvertor::}

Function getDatumEllipsoid( DATUMd:Integer ):String;

begin

If getDatumParameters( DATUMd )then

getDatumEllipsoid:=

WorkDatum.ELLIPSOIDd_ellipsoid

else

getDatumEllipsoid:='';

end;

Function  getDatumParameters( DATUMd:Integer ):Boolean;

{d_datumParams: array[0..23] of DatumParameters;}

begin

WorkDatum.DATUMd_datum:=d_datumParams[DATUMd].DATUMd_datum;

WorkDatum.ELLIPSOIDd_ellipsoid:=d_datumParams[DATUMd].ELLIPSOIDd_ellipsoid;

WorkDatum.d_deltaX:=d_datumParams[DATUMd].d_deltaX;

WorkDatum.d_deltaY:=d_datumParams[DATUMd].d_deltaY;

WorkDatum.d_deltaZ:=d_datumParams[DATUMd].d_deltaZ;

Result:=True;

End;

Procedure FillDatumEllipsoids;

Begin

{d_datumParams: array[0..23] of DatumParameters;}

d_datumParams[0].DATUMd_datum:='ADINDAN';

d_datumParams[0].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_datumParams[0].d_deltaX:=-162;

d_datumParams[0].d_deltaY:=-12;

d_datumParams[0].d_deltaZ:= -206;

d_datumParams[1].DATUMd_datum:='ARC1950';

d_datumParams[1].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_datumParams[1].d_deltaX:=-143;

d_datumParams[1].d_deltaY:=-90;

d_datumParams[1].d_deltaZ:= -294;

d_datumParams[2].DATUMd_datum:='ARC1960';

d_datumParams[2].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_datumParams[2].d_deltaX:=-160;

d_datumParams[2].d_deltaY:=-8;

d_datumParams[2].d_deltaZ:= -300;

d_datumParams[3].DATUMd_datum:='AUSTRALIAN_GEODETIC_1966';

d_datumParams[3].ELLIPSOIDd_ellipsoid:='AUSTRALIAN_NATIONAL';

d_datumParams[3].d_deltaX:=-133;

d_datumParams[3].d_deltaY:=-48;

d_datumParams[3].d_deltaZ:= -148;

d_datumParams[4].DATUMd_datum:='AUSTRALIAN_GEODETIC_1984';

d_datumParams[4].ELLIPSOIDd_ellipsoid:='AUSTRALIAN_NATIONAL';

d_datumParams[4].d_deltaX:=-134;

d_datumParams[4].d_deltaY:=-48;

d_datumParams[4].d_deltaZ:= 149;

d_datumParams[5].DATUMd_datum:='CAMP_AREA_ASTRO';

d_datumParams[5].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[5].d_deltaX:=-104;

d_datumParams[5].d_deltaY:=-129;

d_datumParams[5].d_deltaZ:= 239;

d_datumParams[6].DATUMd_datum:='CAPE';

d_datumParams[6].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_datumParams[6].d_deltaX:=-136;

d_datumParams[6].d_deltaY:=-108;

d_datumParams[6].d_deltaZ:= -292;

d_datumParams[7].DATUMd_datum:='EUROPEAN_DATUM_1950';

d_datumParams[7].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[7].d_deltaX:=-87;

d_datumParams[7].d_deltaY:=-98;

d_datumParams[7].d_deltaZ:= -121;

d_datumParams[8].DATUMd_datum:='EUROPEAN_DATUM_1979';

d_datumParams[8].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[8].d_deltaX:=-86;

d_datumParams[8].d_deltaY:=-98;

d_datumParams[8].d_deltaZ:= -119;

d_datumParams[9].DATUMd_datum:='GEODETIC_DATUM_1949';

d_datumParams[9].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[9].d_deltaX:=84;

d_datumParams[9].d_deltaY:=-22;

d_datumParams[9].d_deltaZ:= 209;

d_datumParams[10].DATUMd_datum:='HONG_KONG_1963';

d_datumParams[10].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[10].d_deltaX:=-156;

d_datumParams[10].d_deltaY:=-271;

d_datumParams[10].d_deltaZ:= -189;

d_datumParams[11].DATUMd_datum:='HU_TZU_SHAN';

d_datumParams[11].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[11].d_deltaX:=-634;

d_datumParams[11].d_deltaY:=-549;

d_datumParams[11].d_deltaZ:= -201;

d_datumParams[12].DATUMd_datum:='INDIAN';

d_datumParams[12].ELLIPSOIDd_ellipsoid:='EVEREST';

d_datumParams[12].d_deltaX:=289;

d_datumParams[12].d_deltaY:=734;

d_datumParams[12].d_deltaZ:= 257;

d_datumParams[13].DATUMd_datum:='NAD27';

d_datumParams[13].ELLIPSOIDd_ellipsoid:='CLARKE_1866';

d_datumParams[13].d_deltaX:=-8;

d_datumParams[13].d_deltaY:=160;

d_datumParams[13].d_deltaZ:= 176;

d_datumParams[14].DATUMd_datum:='NAD83';

d_datumParams[14].ELLIPSOIDd_ellipsoid:='GRS_1980';

d_datumParams[14].d_deltaX:=0;

d_datumParams[14].d_deltaY:=0;

d_datumParams[14].d_deltaZ:= 0;

d_datumParams[15].DATUMd_datum:='OLD_HAWAIIAN_MEAN';

d_datumParams[15].ELLIPSOIDd_ellipsoid:='CLARKE_1866';

d_datumParams[15].d_deltaX:=89;

d_datumParams[15].d_deltaY:=-279;

d_datumParams[15].d_deltaZ:= -183;

d_datumParams[16].DATUMd_datum:='OMAN';

d_datumParams[16].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_datumParams[16].d_deltaX:=-346;

d_datumParams[16].d_deltaY:=-1;

d_datumParams[16].d_deltaZ:= 224;

d_datumParams[17].DATUMd_datum:='ORDNANCE_SURVEY_1936';

d_datumParams[17].ELLIPSOIDd_ellipsoid:='AIRY';

d_datumParams[17].d_deltaX:=375;

d_datumParams[17].d_deltaY:=-111;

d_datumParams[17].d_deltaZ:= 431;

d_datumParams[18].DATUMd_datum:='PUERTO_RICO';

d_datumParams[18].ELLIPSOIDd_ellipsoid:='CLARKE_1866';

d_datumParams[18].d_deltaX:=11;

d_datumParams[18].d_deltaY:=72;

d_datumParams[18].d_deltaZ:= -101;

d_datumParams[19].DATUMd_datum:='PULKOVO_1942';

d_datumParams[19].ELLIPSOIDd_ellipsoid:='KRASSOVSKY';

d_datumParams[19].d_deltaX:=27;

d_datumParams[19].d_deltaY:=-135;

d_datumParams[19].d_deltaZ:= -89;

d_datumParams[20].DATUMd_datum:='PROVISIONAL_S_AMERICAN_1956';

d_datumParams[20].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_datumParams[20].d_deltaX:=-288;

d_datumParams[20].d_deltaY:=175;

d_datumParams[20].d_deltaZ:= -376;

d_datumParams[21].DATUMd_datum:='TOKYO';

d_datumParams[21].ELLIPSOIDd_ellipsoid:='BESSEL';

d_datumParams[21].d_deltaX:=-128;

d_datumParams[21].d_deltaY:=481;

d_datumParams[21].d_deltaZ:= 664;

d_datumParams[22].DATUMd_datum:='WGS_72';

d_datumParams[22].ELLIPSOIDd_ellipsoid:='WGS_72_ELLIPSOID';

d_datumParams[22].d_deltaX:=0;

d_datumParams[22].d_deltaY:=0;

d_datumParams[22].d_deltaZ:= -4.5;

d_datumParams[23].DATUMd_datum:='WGS_84';

d_datumParams[23].ELLIPSOIDd_ellipsoid:='WGS_84_ELLIPSOID';

d_datumParams[23].d_deltaX:=0;

d_datumParams[23].d_deltaY:=0;

d_datumParams[23].d_deltaZ:= 0;

{d_ellipsoidParams: array[0..13] of EllipsoidParameters;}

d_ellipsoidParams[0].ELLIPSOIDd_ellipsoid:='CLARKE_1866';

d_ellipsoidParams[0].d_deltaA:=-69.4;

d_ellipsoidParams[0].d_deltaF:=-0.000037264639;

d_ellipsoidParams[1].ELLIPSOIDd_ellipsoid:='CLARKE_1880';

d_ellipsoidParams[1].d_deltaA:=-112.145;

d_ellipsoidParams[1].d_deltaF:=-0.000054750714;

d_ellipsoidParams[2].ELLIPSOIDd_ellipsoid:='BESSEL';

d_ellipsoidParams[2].d_deltaA:=739.845;

d_ellipsoidParams[2].d_deltaF:=0.000010037483;

d_ellipsoidParams[3].ELLIPSOIDd_ellipsoid:='INTERNATIONAL_1967';

d_ellipsoidParams[3].d_deltaA:=-251.0;

d_ellipsoidParams[3].d_deltaF:=-0.000014192702;

d_ellipsoidParams[4].ELLIPSOIDd_ellipsoid:='WGS_72_ELLIPSOID';

d_ellipsoidParams[4].d_deltaA:=2.0;

d_ellipsoidParams[4].d_deltaF:=0.00000003121057;

d_ellipsoidParams[5].ELLIPSOIDd_ellipsoid:='EVEREST';

d_ellipsoidParams[5].d_deltaA:=860.655;

d_ellipsoidParams[5].d_deltaF:=0.000028361368;

d_ellipsoidParams[6].ELLIPSOIDd_ellipsoid:='GRS_1980';

d_ellipsoidParams[6].d_deltaA:=0.0;

d_ellipsoidParams[6].d_deltaF:=-0.000000000016;

d_ellipsoidParams[7].ELLIPSOIDd_ellipsoid:='AIRY';

d_ellipsoidParams[7].d_deltaA:=573.604;

d_ellipsoidParams[7].d_deltaF:=0.000011960023;

d_ellipsoidParams[8].ELLIPSOIDd_ellipsoid:='MODIFIED_EVEREST';

d_ellipsoidParams[8].d_deltaA:=832.937;

d_ellipsoidParams[8].d_deltaF:=0.000028361368;

d_ellipsoidParams[9].ELLIPSOIDd_ellipsoid:='MODIFIED_AIRY';

d_ellipsoidParams[9].d_deltaA:=796.811;

d_ellipsoidParams[9].d_deltaF:=0.000011960023;

d_ellipsoidParams[10].ELLIPSOIDd_ellipsoid:='WGS_84_ELLIPSOID';

d_ellipsoidParams[10].d_deltaA:=0.0;

d_ellipsoidParams[10].d_deltaF:=0.0;

d_ellipsoidParams[11].ELLIPSOIDd_ellipsoid:='AUSTRALIAN_NATIONAL';

d_ellipsoidParams[11].d_deltaA:=-23.0;

d_ellipsoidParams[11].d_deltaF:=-0.000000081204;

d_ellipsoidParams[12].ELLIPSOIDd_ellipsoid:='KRASSOVSKY';

d_ellipsoidParams[12].d_deltaA:=-108;

d_ellipsoidParams[12].d_deltaF:=0.000000480795;

d_ellipsoidParams[13].ELLIPSOIDd_ellipsoid:='HOUGH';

d_ellipsoidParams[13].d_deltaA:=-133;

d_ellipsoidParams[13].d_deltaF:=-0.000014192702;

End;

end.