(c) Copyright 1992 Genashor Corp.  All Rights Reserved.

The following summary should help you understand the accompanying

network description (.net) files.   The SIMIC User's Guide contains

a complete description of SIMIC's Network Description Language (SNL).

This file also contains descriptions of all SIMIC primitives used in

these demonstration circuits.

Structure Of Circuit Descriptions

---------------------------------

Network topology is specified in the !LOGICAL section of the network

description file. SIMIC supports a general hierarchical network

description capability, where the top-level circuit (the "main

type") and constituent subcircuits (macros) consist of:

(a) a TYPE statement, which specifies the macro's input,

output, and bidirectional (bus) ports and, optionally,

their electrical characteristics, followed by

(b) one or more PART statements, which specify the macro's

internal elements and their interconnections. The internal

elements can either be SIMIC primitives or instances of

other macros.

Normally, each statement is logically terminated by a carriage-

return (or newline). Long statements that require more than one

line may be continued by placing a dollar sign ($) at the end of

each line to be continued.

Comments may be added inside of, and between, macro definitions; they

follow the "COMMENT=" or "C=" keyword.

TYPE And PART Statements Contain Keyword-Fields

-----------------------------------------------

Each TYPE and PART statement contains keyword-fields separated by

whitespace - either spaces or tabs. For example, the TYPE statement

in demonstration file bushold.net is:

Type=bushold  I=EnA,DataA,EnB,DataB  O=E  Olod=1

This statement begins the definition of the circuit.  The first

keyword field, "Type=bushold" specifies	the circuit type (or name)

as "bushold".  The second keyword-field, "I=EnA,DataA,EnB,DataB",

specifies this macro's four input pins; "EnA", "DataA", "EnB", and

"DataB".  The third keyword-field, "O=E", specifies this macro's

single output pin, named "E". The last keyword-field, "Olod=1",

specifies that the load associated with output pin E is 1.

Each PART statement following the TYPE statement describes a component

in the macro. The PART statement assigns the component a unique

instance name within the macro, specifies the component's type (either

a SIMIC primitive or the name of another macro), and names the signals

connected to the component's input, output, and bidirectional (bus)

pins. The PART statement may optionally specify component electrical

characteristics such as output delays, pin loading, and output drive.

For example, the first component of the bushold circuit can be

specified as:

Part=u1  Type=tpadn  I=EnA,DataA  O=Bus  Odel=Del10

The first keyword-field, "Part=u1", assigns the name "u1" to this

component. The second keyword-field, "Type=tpadn", specifies that

this component is a tpadn, a SIMIC primitive. The third keyword-field,

"I=EnA,DataA", specifies the two input signals (which are also

connected to the identically-named macro pins). The third keyword-

field, "O=Bus", specifies that the internal signal named "Bus" is

connected to this component's output. Finally, the keyword-field

"Odel=Del10" specifies that the global delay table named "Del10"

describes the delay characteristics of this output.

!FORMAT Statements Eliminate Entry Of Most Keywords

---------------------------------------------------

Since most PART statements will, with minor exception, contain the

same keywords, it should not be necessary to enter these common keywords

in every PART statement. The !FORMAT statement informs SIMIC of the

keyword-field order in all PART statements that follow it. Keywords

need only be entered for those PART statements that deviate from the

!FORMAT specification.

For example, the statement:

!FORMAT Part=  Type=  I=  O= Odel=

inform SIMIC that all keyword-fields should be expected to occur in

the specified order (first the "Part=" keyword-field, then the

"Type=" keyword-field, etc.). If a subsequent PART statement exactly

utilizes these keyword-fields in this specified order, only the right-

sides of the keyword-fields need be specified. Otherwise, complete

keyword-fields should be specified. Thus, having described this format,

the first component of the bushold circuit can be instantiated with:

u1	tpadn	EnA,DataA	Bus  	Del10

which is equivalent to:

Part=u1  Type=tpadn  I=EnA,DataA  O=Bus  Odel=Del10

PART statements containing keyword-fields other than those specified

in the !FORMAT statement can be handled most conveniently by first

specifying these keyword-fields last. For example, the last component

instantiation in the bushold circuit

u5	and	Bus		E	Del10 Ilod=1

adds the keyword-field "Ilod=1", and is equivalent to the statement

Part=u5  Type=and  I=Bus  O=E  Odel=Del10  Ilod=1

Keywords Used In Demonstration Network Description Files

--------------------------------------------------------

B 		specifies bidirectionals

BLOD 		specifies loading at bidirectionals (default is zero)

I 		specifies inputs

ILOD 		specifies loadint at inputs (default is zero)

O 		specifies outputs

OCHANGE		specifies the value of (identical) rise and fall delays

(constant or load-dependent - default is zero)

ODEL 		specifies delay by referencing global delay table names

ODRIVE 		specifies output drive (POWER-RAIL, DRIVING, RESISTIVE,

or FLOATING -- default is DRIVING)

OFALL 		specifies fall delay (constant or load-dependent

- default is zero)

OLOD 		specifies loading at outputs (default is zero)

ORISE 		specifies rise delay (constant or load-dependent

- default is zero)

PART 		specifies component instance name

TIMING-CHECKS	specifies flip-flop timing checks

TYPE 		specifies macro name in TYPE statements, instance type

in PART statements

SIMIC Primitives Used In Demonstration Network Description Files

----------------------------------------------------------------

AND	AND gate (1 <= number of inputs <= 32767)

BTGN	ideal (resistanceless) switch having two bidirectional pins

and one control input; switch is ON (bidirectional

pins are connected) when the control input is logic-1,

and OFF when the control input is logic-0.

DPCF	positive-edge-triggered D-flip-flop with active-low RESET and

SET asynchronous inputs. input ordering is NR,NS,CL,D.

output ordering is Q,NQ (the NQ output is optional)

INV	inverter

NAND	NAND gate (1 <= number of inputs <= 32767)

NOR	NOR gate (1 <= number of inputs <= 32767)

TNCF	negative-edge-triggered T-flip-flop with active-low RESET and

SET asynchronous inputs. input ordering is NR,NS,CL.

output ordering is Q,NQ (the NQ output is optional)

TPADN	tristating driver with two inputs and one outut. input ordering

is EN,D. when EN is logic-1, the output is assigned the

value of the D input. when EN is logic-0, the output

tristates.

Reserved Signal Names

---------------------

Two signal names are reserved for special signals:

ONE	- logic-1

ZERO	- logic-0