HZComm - FOSSIL implementation for the Heath/Zenith-100 computer

----------------------------------------------------------------

This file desribes the implementation specific features of this

FOSSIL driver.  For information on use and/or implementation of

a FOSSIL driver, see the enclosed FOSSIL.DOC.

This version of HZComm has been tested with Opus Version 0.00

and Seadog Version 4.0.

This TSR program provides complete support for the FOSSIL spec.,

version 3.  It should be run once, upon system boot up.  The

program will try to determine if it has already been installed,

and will not reinstall if it's already in memory.

HZComm and ZPC

--------------

Some care should be taken if you intend to have HZComm and ZPC

resident at the same time.  HZComm should be loaded AFTER ZPC.

This seems to work for all programs that use either utility,

except SETZPC.  The method SETZPC uses to locate ZPC in memory

seems to be confused by the presence of HZComm.  ZPC itself has

the same problem, and will not detect that it is already in

memory if it is reinvoked.  This is a good way to eat up large

chunks of memory in a hurry.  So the moral is, load ZPC then

HZComm.  If you need to configure ZPC, don't load HZComm until

you're done.

HZComm Implementation Notes

---------------------------

HZComm recognizes two serial ports, Serial-A (J1, COM1 to IBM

types), and Serial-B (J2, COM2).  The port number passed to

HZComm should be 0 for Serial-A and 1 for Serial-B.  As of the

current release, only one port can be active at any one time,

and the port number argument is only used for functions 4 and

20.  All other function ignore the port number argument.

Because of a design "quirk" in the 2661 EPCI that the '100 uses

for serial I/O, the DCD input to the computer is basically

worthless.  The HZComm driver assumes that the DCD output from

your modem (normally pin 8) is connected to the DSR input on

the '100 serial port.  This is pin 22 on Serial-A and pin 6 on

Serial-B.  Don't forget that that Serial-A requires swapped pins

2 and 3, as well as other changes.

Function 0 - standard

Function 1 - standard

Function 2 - standard

Function 3 - standard

Function 4 - The control-C flag checking is not implemented.

Function 5 - standard

Function 6 - standard

Function 7 - standard - the timer on the '100 is 100Hz

Function 8 - standard

Function 9 - standard

Function 10 - standard

Function 11 - standard

Function 12 - standard

Function 13 - standard, with caveats:

HZComm actually stuffs the IBM style keycodes in the BIOS

typeahead buffer.  That way Opus Version 0.00 (which expects

IBM keycodes but uses DOS calls to get them) works as

expected.  When this call is made the keycodes are retrieved

from the BIOS buffer and returns them to the caller.  When

a new version of Opus comes out that uses this call, a new

version of HZComm will be released that doesn't mess with

the BIOS at all.  In that case, keyboard entries will ONLY

be available through this call.

Function 14 - standard, with same caveats as Function 13.

Function 15 - standard, with caveats:

XON/XOFF is supported in the driver for output.  If it ever

becomes necessary for HZComm to be able to impose flow

control on input, this function may be revised.  The UART

used in the '100 stops transmitting by itself when CTS is

removed.  RTS would only be needed in the case of input

flow control.  This MAY be implemented in a future release.

Function 16 - standard (I think)

Function 17 - standard

Function 18 - standard

Function 19 - standard, The use of ZANSI is recommended.

Function 20 - standard

The watchdog activation/deactivation is not effected by inits

(function 4) and uninits (function 5).  This is so that a

watchdog may be set before running a program that does not

monitor the serial port.

Function 21 - standard

Function 22 - standard, maximum of 4 routines added to chain.

Function 23 - standard, both "types" of boot are identical

Version History

---------------

03/01/87	Version 2.31

First public release.  Supports all functions required to support

Fido, Opus V0.00 and SEADog V4.0.

03/12/87	Version 3.0

First release of FOSSIL draft version 3 implementation.