Laserjet Setup Utility

version 5.04

Guy Gallo

January 23, 1989

Table of Contents

1. Introduction...........................................1

1.1 Purpose..........................................1

1.2 Modes of operation...............................1

1.2.1 Standard Application Mode.................1

1.2.2 Memory Resident (TSR) Mode................1

1.2.3 Batch Mode................................2

1.3 Requirements.....................................3

1.4 Setting up LSU...................................4

1.4.1 The Mode Command..........................4

1.4.2 Gotta Use the Serial Port,

and Only the Serial Port........................6

1.4.3 Big Warning...............................6

2. Some Notes:............................................9

2.1 Internal fonts...................................9

2.2 Pitch and Points.................................10

2.3 Symbol Sets......................................11

2.4 Styles...........................................12

2.5 Command Selection................................13

2.6 The <Esc> key, the Escape Code:..................14

3. The Program Itself.....................................16

3.1 The Fonts Menu...................................16

3.2 The Symbol Menu..................................17

3.3 The Style Menu...................................17

3.3.1 Bold/Italic/Underline ON/OFF..............17

3.3.2 Alt Bold/Italic ON/OFF....................18

3.4 The Commands Menu................................20

3.4.1 Reset printer.............................20

3.4.2 Eject current page........................20

3.4.3 Manual feed...............................20

3.4.4 Cassette feed.............................20

3.4.5 Wrap lines ON/OFF.........................21

3.4.6 Number of copies..........................21

3.4.7 Printer self test.........................22

3.4.8 Font Example..............................22

3.5 The Layout Menu..................................22

3.5.1 Left offset...............................22

3.5.2 Right limit...............................22

3.5.3 Top margin................................23

3.5.4 Clear margins.............................23

3.5.5 lines per Inch............................23

3.5.6 Text length...............................25

3.5.7 Sixty-six.................................26

3.5.8 Fifty-one.................................26

3.5.9 Eighty-six................................27

3.5.10 Print pitch..............................27

- i -

Table of Contents

3.6 The Envelope Menu................................27

3.6.1 New envelope..............................27

3.6.2 Edit envelope.............................28

3.6.3 Save envelope.............................29

3.6.4 Load envelope.............................29

3.6.5 Business (9x4)............................29

3.6.6 Check    (6x8)............................29

3.6.7 Page    (8x11)............................29

3.6.8 Halfpg   (5x8)............................29

3.7 The Print menu...................................30

3.7.1 Portrait  6 lpi...........................30

3.7.2 Landscape 6 lpi...........................31

3.7.3 porTrait  8 lpi...........................31

3.7.4 landScape 8 lpi...........................31

3.7.5 Directory.................................31

3.8 The Options Menu.................................32

3.8.1 LSU 5.04..................................32

3.8.2 Unload LSU................................32

3.8.3 Hotkey:  Alt-F10..........................32

3.8.4 Force mono YES/NO.........................33

3.8.5 Model I/II................................33

3.8.6 Customize fonts...........................33

3.8.7 Port LPT1/COM1............................35

3.8.8 Save defaults.............................35

4. Warranties and things..................................37

- ii -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_______________           1. Introduction

___________              1.1 Purpose

Laserjet Setup Utility functions as either:

1) a utility to set various parameters on

the Laserjet family of printers so that

software which does not fully address these

printers can be made to use fonts and

functions that would otherwise be

inaccessible, or

2) as a stand alone program to print ASCII

text files, and to address envelopes.

______________________              1.2 Modes of operation

Laserjet Setup Utility can be run in any of three modes:

_______________________________               1.2.1 Standard Application Mode

As a standard DOS application, invoked from the DOS

command line:

Example:

C:>LSU <Enter>

In this mode, the program loads, selected options are sent

to the printer, and the program is exited.  The selected

options remain in effect until reset either by a

subsequent selection via LSU, or by a subsequently run

program which addresses the Laserjet directly.

- 1 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

________________________________               1.2.2 Memory Resident (TSR) Mode

When executed with the "/L" (as in "Load") parameter, LSU

will run as a Terminate and Stay Resident (TSR) program.

Example:

C:>LSU /L <Enter>

A copyright notice is printed to the screen, and the

current Hotkey is displayed.  The Hotkey is that

combination of keystrokes which will invoke LSU from

within another application.

The default Hotkey is Alt-F10.  However, the Hotkey can be

changed and saved to disk.  How to do so, and how to save

the selected Hotkey permanently, will be described later

in this manual (see Options.Hotkey, section 3.8.3).

Options selected will remain in effect until reset by a

second accessing of LSU or by a second application

program.

NOTE:  TSR mode is not available if your

application places the computer in

"graphics mode".  If you inadvertently call

up LSU over a graphics program, the

computer will seem to freeze.  Pressing Esc

(and exiting the invisible LSU) should

rectify things.

________________               1.2.3 Batch Mode

As a single function batch command, LSU can be used to

send a user defined code to the printer from the command

line or from within a batch file.

- 2 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Batch mode is useful if you have a specific command which

you are regularly going to send to the Laserjet.  Batch

mode allows you to do so without having to interact with

the menu interface.

Batch mode is invoked by the parameter "/B" (which stands

for "Batch").  Case is insignificant.

Examples:

From the command line:

LSU /B <Enter>

From within a batch file:

@Echo off

rem this is my Autoexec.bat

DATE

TIME

MODE COM1 9600,n,8,1,[p]

MODE LPT1 = COM1

LSU /B  <--  [this invokes LSU in batch

mode code]

CD\MyWPDir

MYWP

The code which is sent to the printer when LSU is loaded

in batch mode is called the Custom/Auto code (because it

is customizable by the user, and because it is

automatically sent...).

NOTE:  by default the Custom/Auto Code sets

the Laserjet to a printable page of 66

lines.  The Custom/Auto code, which is the

last item on the Fonts menu, can be edited

and saved to disk.  (See Options.Save

Defaults, section 3.8.8).

- 3 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

________________              1.3 Requirements

--  An IBM PC, PS/2 or close compatible computer.

--  128K of available memory for Standard and Batch modes.

--  60K memory over and above memory needed for running

your application if you intend to use LSU in TSR mode.

--  The Hewlett Packard Laserjet, Laserjet Plus, or

Laserjet Model II.

--  Any additional font cartridges.

--  Paper and a little patience.

__________________              1.4 Setting up LSU

By default, LSU uses the parallel port to communicate with

the printer.  This means that if you have 1) a Standard

Laserjet or a Laserjet Plus, or if you have 2) your Model

II/IID connected to your computer's serial port, you must

configure your system in one of two ways to accommodate

this:

1) Either issue the MODE command to

initialize the serial port followed by the

MODE command to redirect all output from

the parallel port to the serial port, or

2) Issue the MODE command to initialize the

serial port and configure LSU to address

the Laserjet via the serial port directly

(see Options.Port, section 3.8.7)

- 4 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

______________________               1.4.1 The Mode Command

MODE is an external "command" (a small utility program)

that comes with DOS.  If you have never used a serial

printer, then it is likely you have never had need of this

little utility.  But, now you do.

Among other things, MODE initializes the serial port,

preparing it for any kind of communication.  Most

telecommunications programs take care of this

initialization themselves.  But to use a printer, you have

to do so manually.

Initialization has three, or, optionally, four parts.

The format for running MODE is:

MODE [port]:[speed][parity][databits][stopbits][P]

The meaning of all these arcane terms can be found in your

DOS manual.

Laserjets require the following setting (you should place

it in your AUTOEXEC.BAT file):

MODE COM1 9600,n,8,1,[p]

This initializes your serial port.  Which translates into:

"Please tell my Laserjet that I will be sending data at

9600 baud, with no parity checking, 8 data bits, with 1

stop bit, and if I get a time-out error, I will continue

to try until something gives."  This last part of the

command, the "p", is not required for LSU to work, but

*is* required for many programs.  Like Microsoft Word.

So, you might as well include it.  Doing so will cost you

about 368 bytes of memory.

But that's not the end of it.  Since LSU uses the parallel

port by default, if you have a Standard Laserjet or a

- 5 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Model II connected to the serial port, you *also* must

issue the following command.

MODE LPT1 = COM1

This "redirects" output that is sent to the parallel to

the serial port.

Any program which would normally only print to the

parallel port will now work with your serially connected

printer.

Refer to your DOS manual for a more detailed explanation

of the MODE command.

The Laserjet Model II can be configured either as a

parallel or a serial printer.  If you have a Model II

connected to your serial port, follow the above

instructions.  If your Laserjet is connected to your

parallel port, no special initialization is required.

_________________________________________________________               1.4.2 Gotta Use the Serial Port, and Only the Serial Port

If, for whatever reason, you *must* use the serial port

only -- that is, the "parallel redirection" trick outlined

above won't work for you -- then you can tell LSU to

address the printer directly through the serial port.

You select the port via the Options.Port command (see

below, section 3.8.7).  Your preference can be saved to

disk with the Options.Save defaults command.

_________________               1.4.3 Big Warning

LSU has gotten slightly smarter with version 5.04.  The

program now can test whether a Laserjet, which is

connected to a parallel port, is ready to receive input.

- 6 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

If you have a Laserjet Model II or later, connected to the

parallel port, then you will get a polite error message if

the printer is Off, or Off-Line, or Out of Paper.

However.  There are still some precautions to take if you

have a Laserjet I (which was a serial printer).

Please be aware that LSU (at this time) has no way to know

if you have correctly configured a serial printer.

If you try to send a code directly the serial port without

having sent the appropriate MODE command, you will get a

constantly flashing "40" on the status indicator.  It

won't go away until you turn off your Laserjet or you

empty the printer buffer by placing the printer Off-line

and executing a Form Feed.  This is benign enough, and you

probably won't lose any data, even if you are using LSU in

TSR mode while writing a terrific novel.

HOWEVER:  if you try to send a code through the parallel

port and you have failed to BOTH initialize the serial

port properly, AND redirect all output from LPT1 to COM1,

then you are in for a big surprise.  Your computer will

lock up and you'll need to reboot (Ctrl-Alt-Del).  This is

benign enough if you were just running LSU, but if you

have data stored on a ram disk, or if you are using LSU in

TSR mode and are in the middle of doing your taxes, you

won't be very happy.

If you have a serial printer PLEASE take care to write one

or both of the following lines into your AUTOEXEC.BAT

file:

MODE COM1 9600,n,8,1,[p]  (initialize)

MODE LPT1 = COM1          (redirect)

The safest bet, even if you will be addressing the

Laserjet directly through the serial port when using LSU,

is to include both lines.  It can't hurt.  And you will

eventually come across a program that can only print

through the parallel port (in which case the redirection

line will be necessary.

- 7 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

If you are not addressing a serial Lasrjet directly

through COM1, and you have entered the above MODE

commands, then your serial printer will act just like a

parallel printer when there is an error condition.  That

is, you will also get the error messages stating that

there is a "Printer not ready" error.

By the way, it is possible for LSU to *seem* to crash when

it hasn't if the On-Line indicator is not illuminated.

Before you reach for the Big Red Switch, please check that

the printer is in fact on, and on-line.

- 8 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

______________           2. Some Notes:

__________________              2.1 Internal fonts

LSU does not create fonts.  That is, even if a font is

visible as an option on the Fonts menu, the availability

of that font depends upon which cartridges you own, and

which cartridge is loaded into the printer.

Each of the Laserjet models have several internal fonts

built into the machine.  These fonts are available no

matter what other cartridges you own and have loaded.

The internal fonts built into the Laserjet printers are as

follows:

Standard Laserjet:

Courier 10 portrait

Courier 10 landscape

Laserjet Plus and Laserjet 500 Plus

Courier 10 portrait

Courier 10 landscape

Line Printer 16.66 portrait

Laserjet II

Courier 10 portrait, + bold

Courier 10 landscape, + bold

Line Printer 16.66 portrait

Line Printer 16.66 landscape

Laserjet IID - (can rotate fonts, so all fonts are

available in Portrait and Landscape):

- 9 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Courier 10  + bold, italic

Courier 12, + bold, italic

Line Printer 16.66

The S2 font cartridge is bundled with the IID, so you

could consider the following built in:

Times Roman 12 pt, + bold, italics,

Times Roman 8 pt

Helvetica 14 pt bold

If you wish to access additional fonts, such as Times

Roman, or Helvetica, or Letter Gothic, you must purchase

the appropriate font cartridge.

LSU's default configuration contains font selection

information for several HP font cartridges.  Further, the

Font menu can be customized to suit the cartridges you

possess.

NOTE:  if you select a font from LSU's Font

menu for which you do not actually possess

the appropriate font cartridge, LSU will

send the code to the printer;  the Laserjet

will attempt to set the printer to the

available font that most closely matches

your choice.

Example:

If you have the Times Roman B cartridge

(which only has Times Roman in 8 and 10

point), and you select Times Roman 12 point

-- you will in fact get Times Roman 10

point.

____________________              2.2 Pitch and Points

One of the great virtues of laser printing is the ability

to format documents with different fonts.  A font is a

mixture of several qualities:

- 10 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

With a fixed font each character is the same width.  As a

result you can calculate line length by the number of

inches to a line, and the number of characters to an inch.

This is identical to a typewriter.  For example, one of

the most common fonts (both in Laserjets and on

typewriters) is Courier 10.  The "10" here refers to Pitch

-- that is, Courier 10 prints 10 characters per inch.

Prestige Elite 12 prints 12 characters per inch.  Line

Printer 16.66 prints 16.66 character per inch.

Proportional fonts cannot be described in terms of pitch,

since each character has a different width (for example,

"i" is much narrower than "a").  Therefore, the size of a

proportionally spaced font is described by it's height in

points.  One point is equal to approximately 1/72th of an

inch.

Pitch should not be confused with Points.  Pitch refers to

how many characters fit within an inch.  Point size refers

to the height of the font measured in 1/72nds of an inch.

Some confusion arises because fixed fonts are often de-

scribed using both measurements.  So, you have Courier 10p

12pt (10 pitch 12 point) and Prestige Elite 12p 10pt.  The

fact that the numbers are identical, but reversed can eas-

ily befuddle.  Just remember, pitch is the most meaningful

description of a fixed font, and point size is the most

meaningful description of proportionally spaced fonts.

For the purposes of LSU and its documentation, pitch will

be represented by a single 'p', and point size by the

abbreviation 'pt'.

_______________              2.3 Symbol Sets

Another aspect of HP Laserjet fonts that sometimes creates

confusion is Symbol Set.

Most fonts default to the Symbol Set called Roman-8.  The

Internal fonts are Roman-8.  But on some cartridges, you

have a choice of Symbol Sets.

- 11 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

The reason for selecting an alternative Symbol Set has to

do with the "extended" characters.  For example, what

appears when you hit the "~" (tilde) or the "'" (single-

quote) key?  Or, what foreign characters are available?

USASCII and Roman Extension have the alphabet with accent

marks mapped into the extended range (160-255).

On the HP92286Z cartridge, there are two possible symbol

sets, ASCII and ECMA-94 Latin 1, 7 bit.  Neither of these

symbol sets have the extended foreign language characters,

but their difference might be instructive about symbol

sets in general.

The Fonts menu has a pre-determined Symbol Set, formulated

according to my own, limited, font cartridges.  If you

have access to other Symbol Sets, you have two options:

edit the Font menu (see Options.Customize fonts, section

3.8.6), or select the alternate Symbol Set from the

Symbols menu *after* you have selected the font.

For example:

If wish to use Helv 10 pt in EMCA, you must do so in two

steps:

1) select option "J" from the Fonts menu.

This will set Helv 10pt USASCII.

2) select EMCA from the Symbol Menu

Alternatively, you could configure option "J" on the Fonts

Menu to *always* set EMCA by editing the code sent to the

printer.  (Options.Customize fonts, section 3.8.6)

__________              2.4 Styles

Underlining, though it definitely changes the look of a

document, is not considered a change in the font.  All

fonts can be underlined.  Bold and italic, however,

actually change the look of the font itself.

- 12 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

The availability of bold and italic depends upon the model

of your Laserjet, and the active cartridge.

_____________________              2.5 Command Selection

Selecting a main menu item and pressing <Enter> invokes a

sub-menu.  Alternately, pressing the first capitalized

letter of the main menu item will access the appropriate

sub-menu.

For example:

At the main menu, pressing 'F' will bring

down the Fonts menu.

Pressing 'S' will bring down the Symbol

menu.

Pressing 'T' will bring down the sTye menu.

Similarly, menu items can be accessed either by selecting

the item with the cursor movement keys and pressing

<Enter> or by selecting the *first* capitalized letter of

the appropriate selection.

Example:

The first Font menu selection is "A:

Courier 10p USASCII [A,C,H,L,Q,Y]"  This

item can be selected (and the appropriate

code sent to the printer) either by

highlighting and pressing <Enter>, or by

pressing the 'A' key.  Note that although

there are many capitalized letters in this

menu selection, 'A' is the first.

The Esc key is an all purpose exit key.  It exits any

active menu or prompt, exits the envelope editor, and,

when pressed from the main menu, exits LSU.

- 13 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

___________________________________              2.6 The <Esc> key, the Escape Code:

Pressing <Esc> from any selection prompt aborts the

command; from any sub-menu it returns to the main menu;

from the main menu it Quits the program.

This key (as it actually exists on your keyboard, and as

it is used to "escape" from a command in progress) should

not be confused with the Escape Code -- a term you may

have found in your HP Manual.

Almost all printers, including the HP Laserjets,

communicate with programs via a series of codes -- and the

way in which the computer tells the printer that the

following series of characters are *not* to be printed,

and *are* to be taken as formatting commands, is to

preface the command string with a very special character,

which, unfortunately, is called "Escape".  You would think

that for a character so often used, they (those people who

invented keyboards and computer codes) could have found

two discreet names for the physical Key and the character.

Anyway...

This character is usually represented in texts as either

Ec, or <ESC>, or Esc, and is variously called Decimal 27,

Hexadecimal 1b, or just plain "escape".  In an actual disk

file (for instance, if you peek into the program file

LSU.COM), this character looks like a small left pointing

arrow.  That's what it looks like on LSU's Customize fonts

screen.

But how do you enter this mysterious key?

Well, this is often a source of confusion (and one of the

reasons setup programs like LSU are necessary).  The

answer is simple.  And the answer is necessary if you are

going to customize LSU's Fonts menu.  The answer, which I

will repeat in the section on customizing LSU, is:  hold

down the Ctrl key with your left pinky, and press (once)

the LeftBracket key ('[').  In the remainder of this

document, and in LSU itself, this key, the Escape Key, is

referred to as '^[' or as Ctrl-LeftBracket.

- 14 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

In summary, you are being asked two very different things

when a program says "Press <Esc> to cancel", and when it

asks you to enter the escape code.  The former is the

physical key marked "Esc" on your keyboard;  the latter is

the escape code (^[, 1bh, #27).

- 15 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_____________________           3. The Program Itself

__________________              3.1 The Fonts Menu

When invoked, the Fonts menu presents the following:

A: Courier 10p  Roman-8 [A,C,H,L,Q,Y]

B: P.Elite 12p  Roman-8 [A,C,H,L,Q,Y]

C: LtrGoth 12p  USASCII [E,N,Q]

D: LinePR  16p  Roman-8 [C,F,L,Y]

E: TmsRmn   8pt USASCII [B,F,K,S2,Z]

F: TmsRmn  10pt USASCII [B,F,K,P,Z]

G: TmsRmn  12pt USASCII [S2,Z]

H: TmsRmn  14pt USASCII [Z]

I: Helv     8pt USASCII [B,F,K,S2,Z]

J: Helv    10pt USASCII [B,F,K,P,Z]

K: Helv    12pt USASCII [S2,Z]

L: Helv    14pt USASCII [Z]

M: C.Land  10p  Roman-8 [internal]

N: LP.Land 16p  Roman-8 [[A,B,C,L,Y,Z]

O: Landscape current font

P: Portrait current font

Q: Send the custom/auto code

Entries A through N present four pieces of information:

The short cut key:         e.g. A

The font name:             e.g. Courier

The font size              e.g. 10p

The symbol set:            e.g. Roman-8

A listing of the

font cartridges:           e.g.

[A,C,H,L,Q,Y]

Selecting any of these choices (either by pressing the

shortcut key or highlighting the selection and pressing

<Enter>) will send that code to the printer immediately,

and exit the menu.

If you do not have one of the cartridges listed within the

brackets, the font which most closely approximates your

choice will be loaded.

- 16 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Entries O and P select the orientation for the font as

Landscape or Portrait.  If the currently selected font

does not have a landscape mode, the printer will usually

default to Courier 10p landscape.

Entry Q holds the Custom/Auto code.  This is the code that

will automatically be sent to the printer when LSU is

invoked with the "/B" parameter.  By default this code

sets the page layout for 66 lines per page (the same as

the Layout.Sixty-six lines command).

NOTE:  all of the entries in the Fonts menu

can be customized.  Both the menu text and

the code sent to the printer can be changed

to suit the fonts you most regularly use.

(See Options.Customize fonts, section 3.8.6

___________________              3.2 The Symbol Menu

This menu allows you to over-ride whatever symbol set you

have defined (or whatever default you have accepted) in

the Fonts menu for the symbol set of a given font.

Be sure that you select this option *after* setting the

desired font using the Fonts menu.

Any choice made  under the Symbols menu will be overridden

by a subsequent selection from the Fonts menu.

__________________              3.3 The Style Menu

__________________________________               3.3.1 Bold/Italic/Underline ON/OFF

This menu allows you to toggle on and off several special

effects.

When you select Bold, Underline, or Italic, any subsequent

print to the Laserjet will be formatted *entirely* with

that style.

- 17 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Selecting Bold and/or Italic has no effect if the fonts

currently available to the printer do not support these

styles.

Underline, however, is available for all fonts, including

landscape.

____________________________               3.3.2 Alt Bold/Italic ON/OFF

You may have noticed in your HP Laserjet manual a lot of

talk about Primary and Secondary fonts, about Shift In and

Shift Out.  I would venture to say that you ignored it,

either because it was confusing (which it is) or because

it never proved useful (which, for the most part, it

isn't).

Well, I'm here to tell you what it is, and give you

something to do with it.

Simply put, by shifting the direction of the parenthesis

in any font description code you tell the Laserjet to

alter one of two fonts.  For example

<Esc>(0U<Esc>(s1p14.4v0s1b4T

This sets the Laserjet's Primary font to (in order,

reading from left to right):  USASCII, Proportionally

Spaced, 14.4pt, Upright, Medium Stroke Weight, Helvetica

(not having to remember all these things is what LSU is

for...).  While:

<Esc>)0U<Esc>)s1p14.4v0s1b4T

This sets the Secondary font to the same thing.  Notice

the left close parenthesis instead of open parenthesis.

So what's the big deal?  Well, theoretically, once you

have set both a Primary and Secondary Font, you can easily

switch between the two by issuing a single Control Code.

- 18 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

That's the Shift In (read from Primary) and Shift Out

(read from Secondary) commands.

This would allow you, for instance, to create a document

with any old editor or word processor, and as long as you

can insert the control codes ^N and ^O (ascii 014 and

015), you could toggle from one to the other.

So you could have Primary as Upright (normal) and

Secondary as Italic, and when you wanted to italicize, you

would write the following:

This next ^NWord^O was italicized.

As it turns out, no word processor (that I know of) uses

this method for formatting text.  The Secondary Font has

been universally ignored.  Until now.

On the sTyle menu you will see two entries, alt bOld OFF

and alt iTalic OFF.  If you toggle either or both of these

to on, then the Secondary Font will be set to Bold and/or

Italic.

Why?  Well, LSU has a built in envelope editor (described

in detail below, section 3.6).  Using ^N and ^O you can do

some fancy formatting of your envelopes, if you have the

necessary landscape fonts loaded.

The envelope editor will accept ^N and ^O as characters.

They are entered by holding down the Ctrl key and tapping

"N" or "O".

If you wanted to get even fancier, you could enter entire

control strings.  For example, you could enter the code

for underline.  To see an example of such formatting, load

LSU and select Envelope.New Envelope.  You will be

presented with an envelope form which contains my return

address formatted with the first line using the Secondary

font, and the second line using underline.

- 19 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_____________________              3.4 The Commands Menu

___________________               3.4.1 Reset printer_

This command clears all settings from the Laserjet's

memory and returns it to it's default status (as if it had

just been turned on).  This usually means:  no margins,

casette feed, and Courier 10p 12pt as the active font.

NOTE:  The exception is when you have a

font loaded, such as Prestige Elite, or

Letter Gothic, whose cartridge default font

overrides the built in default font.  The

cartridge information sheet specifies

whether or not the cartridge fonts over-

ride the internal fonts as default.

________________________               3.4.2 Eject current page

This sends a Form Feed command to the Laserjet.  This is

useful at those times when an application (for whatever

mysterious computer reason) sends a partial page to the

printer, or a left-over character.  You can tell if this

is the case if the Form Feed indicator is illuminated on

the control panel.

_________________               3.4.3 Manual feed

Activates manual feed (this can also be done manually by

placing the printer off-line and pressing the "Manu Feed"

indicator on the Standard Laserjet or by using the Menu

function on the Model II/IID).

This is useful if you are printing letter head stationary,

labels, postcards, etc.

- 20 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

___________________               3.4.4 Cassette feed

Activates casette feed.  Paper will be feed through the

Laserjet from the currently loaded paper tray.

This is the default, and can also be activated by toggling

the "Manu Feed" indicator off.

_______________________               3.4.5 Wrap lines ON/OFF

This command is a toggle.

When it is set to ON:

Lines that extend into the non-printing area will be

wrapped to the next line.

This command is useful for making sure that all text is

printed.

However, if it proves necessary (that is, you have lines

in your text file that are too wide for the selected font)

it usually means you will adversely affect the pagination.

When set to OFF:

Lines that extend into the non-printing area will be cut

short.  This is the default setting.

If lines extend into the non-printing area of the page,

the optimum fix is to re-edit the file into shorter lines,

or select a smaller font before printing.

- 21 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

______________________               3.4.6 Number of copies_

Select the number of copies to print of the next document.

Whether you are printing through LSU, or through your word

processor, this command tells the Laserjet to print x

number of copies.

_______________________               3.4.7 Printer self test

This command sends the Laserjet into test mode.  A single

sheet filled with diagonally shifted characters will be

printed, a series of internal tests will be performed, and

any error conditions will be reported on the front panel

indicator.  A key to the possible error codes can be found

in the Laserjet manual.

This is useful for examining uniformity of print quality,

and as precautionary maintenance.

__________________               3.4.8 Font Example_

Prints a short test at the current settings.  This shows

font, symbol set, orientation, style, margins, pitch, etc.

___________________              3.5 The Layout Menu

_________________               3.5.1 Left offset

Sets the beginning column of the left margin offset.  The

measurement depends upon the currently selected pitch.

For example, entering 10 while Courier 10p is active would

give you a left margin of 1";  while a setting of 12 would

be needed for a 1" offset if Prestige Elite 12p were the

active font.

- 22 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_________________               3.5.2 Right limit_

Sets the column number beyond which characters will not

print.

In effect, this setting, minus the left offset, is equal

to the number of characters per line when you are using a

fixed font.

For example, a left margin of ten and a right margin of 80

would allow for 70 characters per line.

________________               3.5.3 Top margin_

Sets the number of lines to skip from the top of the page;

uses the currently selected number of lines per inch.

___________________               3.5.4 Clear margins_

Clears all margin settings, resetting them to the Laserjet

defaults.

____________________               3.5.5 lines per Inch

This command alters the spacing between lines, and

therefore alters the number of lines per inch the Laserjet

will print.  The default line spacing is 6 lines per inch.

However, with some fonts, such as Prestige Elite, or Line

Printer, you can squeeze more lines into an inch (and

therefore more lines onto the page), without sacrificing

legibility.

This command uses the Vertical Motion Index (see HP

Manual) to set the line spacing.  It tells the Laserjet

how many 1/48th of an inch increments to skip between

lines.

- 23 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

The formula to calculate the VMI is:

(48/x)*(1/48) where x is the number you enter into LSU.

For example, if you were to enter 8, LSU would calculate

the following:

48/8*1/48 = 6/48 -- this would tell the Laserjet to skip

6/48ths of an inch (or 1/8th inch) between each line.

A more complicated example would be setting LPI to 9:

48/9*1/48 = 5.3333/48 -- this would tell the Laserjet to

skip 5.3333 48ths of an inch between each line.

Most frequently, you will use even numbers, such as 3, 6,

8, 12.  But the four decimal places are there if you need

them.

However, just setting the number of lines per inch will

not format a page of text properly.  You still must tell

the Laserjet how many lines constitute a page.

If you are using the default 6 LPI, then the Laserjet will

print approximately 60 lines per page.  This number

changes, however, when you change the LPI.

If you are using a standard DOS application, such as

WordStar or any other program in which you can specify the

number of lines per page, you must adjust the page length

(in the program) to accommodate the increased number of

lines per inch.  The printable page area on a Laserjet

using 8.5 x 11 paper is approximately 10 inches.

Therefore, you can calculate the number of lines per page

by using the formula:

10 * LPI.

For example

- 24 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

At 8 LPI you would be able to print 80

lines, including, of course, top and bottom

margins.  So, if you had WordStar set to

produce a top and bottom margin of 5 lines

each, with a .PL of 80 the actual number of

text lines would be 70.

If, however, you are printing a file

directly from DOS, using the command COPY

FILE.TXT PRN <Enter>, then you must

directly change the number of lines the

Laserjet will print on a page before

executing a form feed.  That is done with

the Text Length Command.

_________________               3.5.6 Text length_

Specifies the page length in number of lines (using the

currently selected line spacing) printable on a single

page.  It is used in conjunction with Top Margin, Bottom

Margin, and LPI.

This command, in combination with the lines per inch

command, will determine the exact size of the printable

area of a page, using the following formula:

(lines per inch) * (x inches per page) - (top margin +

bottom margin)

For example:

If you had a text file captured from an on-line service

such as CompuServe -- a file that has no formatting, no

form feeds, just one line after another, you could format

it rather nicely by following these steps:

- 25 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Set left margin offset to, say, 10.

Set top and bottom margin to 5.

Set LPI to 8.

Set Text Length to 70, (80 - (5+5).

Exit LSU

Execute the command COPY CAPTURE.TXT PRN

<Enter>.

_______________               3.5.7 Sixty-six__

Most PC-DOS programs assume that a printer is capable of

printing 66 lines (6 lines per inch, 11 inches per page).

This is not the case on the Laserjet family, since there

is a border around a page of "unprintable space"  As a

result the Laserjet prints approximately 60 lines per

page.

This command sets the Laserjet to print 66 lines per page

by creating slightly larger than normal top and bottom

margins, and slightly smaller vertical increments between

lines.

This command is extremely useful when using PC-DOS

programs that do not know how to manipulate the Laserjet.

Simply execute the Layout.Sixty-six lines command before

running such a program, and all output should be properly

formatted.

_______________               3.5.8 Fifty-one__

Simply sending a normal ASCII text file to the Laserjet in

landscape mode will result in erratic pagination:  the

Laserjet still expect to print 60 lines per page.  As a

result, page one will print fifty-odd lines, the second

page will print nine lines, and one line (more or less)

will be lost into the never never land of "unprintable

space".

This command tells the Laserjet to expect precisely 51

lines per page.

- 26 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

This command is *necessary* if you are printing a file in

landscape mode from the DOS command prompt using COPY

FILENAME.TXT PRN <Enter>.

________________               3.5.9 Eighty-six_

Similarly, if you have legal sized (8.5 x 14) paper loaded

into the Laserjet, it will still, by default, expect only

60 lines per page.

This command sets the Laserjet to print 86 lines per page

__________________               3.5.10 Print pitch_

Sets the print pitch.  This setting uses Horizontal Cursor

Spacing to determine the fraction of an inch skipped

between characters, and overrides the normal pitch for the

selected font.  This allows you to print Courier at 12

pitch, or Elite at 10 pitch.  Enter the pitch - the number

of characters per inch desired.

This command uses the formula 120/x*1/120 to tell the

Laserjet precisely how many 1/120ths of an inch to skip

between characters.

NOTE:  when using proportional space fonts,

only the SPACE character is affected by the

Pitch command.  The result is that the

*characters* are no closer together (or

further apart), but the *words* are.

_____________________              3.6 The Envelope Menu

__________________               3.6.1 New envelope

Loads a blank envelope with the default Return Address

already inserted.

- 27 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

There are five lines for Return Address and five lines for

Addressee

The envelope remains in memory.  You can then save it to a

file, print it, or return to it with the Edit envelope

command.

A default Return Address is built into LSU.  When you

first run LSU, that default address is mine.  You can save

a new default by entering the Return Address you wish

saved, then executing the Options.Save defaults command.

NOTE:  If you do not want a default return

address to be inserted with each run of New

Envelope, then simply edit out the return

address and Save defaults.

Usage:

First envelope:  this command will load a blank form, with

only the Return Address portion filled in.

After editing an envelope, and changing the Return Address

and/or Addressee, selecting New will remove the edited

envelope from memory and replace it with the default

format (either completely blank, or blank addressee and

default return address...).

___________________               3.6.2 Edit envelope

If you have loaded an envelope from disk, or made changes

to the default envelope, executing Edit envelope allows

you to edit that envelope (that is, without defaulting to

the standard Return Address).

If you execute Edit envelope *first*, that is, before

selecting New -- then you will be presented with a

completely blank form (no default return address).

- 28 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

___________________               3.6.3 Save envelope

Saves the envelope currently in memory to disk.  You will

be warned if the filename given already exists on disk.

It is recommended that you use a consistent file

extension, such as ENV for all envelope files.

___________________               3.6.4 Load envelope

Loads a previously edited and saved envelope file.  You

will be prompted for a filename.  Paths are recognized.

____________________               3.6.5 Business (9x4)

Prints the envelope currently in memory on a business-

sized envelope.

____________________               3.6.6 Check    (6x8)

Prints the envelope currently in memory on a small, check-

sized envelope.

____________________               3.6.7 Page    (8x11)

Prints the envelope currently in memory on a page-sized

envelope

WARNING:  printing manilla envelopes that

have metal tabs on the closure will most

likely result in severe paper jams.  Not

recommended.)

- 29 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

____________________               3.6.8 Halfpg   (5x8)

Prints the envelope currently in memory on a half-page-

sized envelope.

__________________              3.7 The Print menu

Printing a file through Laserjet

The Print menu allows you to print an ASCII document file

to the Laserjet in either portrait or landscape with page

numbers and optional header.

When prompted for Header Text, pressing <Enter> with no

text will disable header printing.  Page numbers will

still be printed.

Note:  The Landscape option of the Print

command sets the right hand margin (line

length) to 160.  This enables you to take

advantage of the Line Printer Font (16.6p)

on cartridge 92286B.  If, however, you have

only the internal (ROM) landscape font

(Courier 10p), be aware that any characters

beyond 80 (minus left margin) will not be

printed.

The left margin is left at zero for both portrait and

landscape under the Print command.  You must set the left

margin manually under Layout.Left Margin in order for the

text file to be formatted properly.

Font selections and margins in effect for a print job will

remain afterwards, LPI chosen by Print will also remain in

effect after printing.

This is true *unless* the print job was executed in either

of the landscape modes.  In those cases, the printer will

be reset to power-on defaults.

- 30 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_____________________               3.7.1 Portrait  6 lpi

Prompts for filename, header, and prints a text file in

portrait orientation, six lines per inch

In portrait mode, 6 lpi will allow 58 text lines.

_____________________               3.7.2 Landscape 6 lpi

In landscape mode, 6 lpi will allow 46 text lines.

A complete RESET will be sent to the printer after

printing.

_____________________               3.7.3 porTrait  8 lpi

In Portrait mode, 8 lpi will allow 78 text lines.

_____________________               3.7.4 landScape 8 lpi

In Landscape mode, 8 lpi will allow 58 text lines.

A complete RESET will be sent to the printer after

printing.

_______________               3.7.5 Directory

Shows a directory according to DOS command line

conventions.

For example, entering

- 31 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

*.doc - shows all files in current directory with file

extensions of doc.

e:\dir\*.bak - shows all files on drive E:, directory

\DIR, with file extensions of BAK.

____________________              3.8 The Options Menu

______________               3.8.1 LSU 5.04

Displays address for license registration of LSU.

________________               3.8.2 Unload LSU

If LSU is in TSR mode, this command will remove LSU from

memory, and return used memory to other applications.

WARNING:  this must ONLY be invoked from

the DOS command line.  If you invoke this

command (and answer yes to the Continue?

question) while you are inside an

application program, you will be dumped to

DOS and any unsaved work will be lost.

In order for this command to return memory to DOS for use

by other applications, LSU *must* have been the last TSR

program loaded.

______________________               3.8.3 Hotkey:  Alt-F10

Shows the current Hotkey, and allows you to change it.

Valid Hotkey possibilities are:

- 32 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

Alt   +  F1 - F10,

Shift +  F1 - F10,

Ctrl  +  F1 - F10,

Alt   +  1 - 0, -, =

_______________________               3.8.4 Force mono YES/NO

Displays the current setting for whether or not you want

LSU to use monochrome color settings despite the fact that

you have a color monitor, because LSU *thinks* you have a

color monitor when in fact you are using a monochrome

graphics screen.

Laserjet can only determine whether or not you have a

Graphics adaptor.  If you have a graphics adaptor, LSU

assumes it is a color adaptor and sets the colors

accordingly.  However, if you are using a laptop with an

LCD or a monochrome graphics adaptor, these color setting

may not be legible.

To remedy this, simply select this option, save the new

default to disk, and re-load LSU.

________________               3.8.5 Model I/II

This is a toggle -- it allows you to set the current model

of your Laserjet.  Possible settings are I and II.  If you

have a Laserjet Plus or Plus 500, leave the setting at I.

If you have a Laserjet IID, use the II setting.

This setting allows LSU to properly position envelopes for

printing.

_____________________               3.8.6 Customize fonts_

This selection allows you to change the functioning of the

Fonts menu.  You will be presented with a screen showing

- 33 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

both the menu text and the associated printer control

code.

Editing Menu Text

When entering a new menu text be aware that the shortcut

key (that is, the key you can press to execute the command

rather than highlighting the command and pressing <Enter>)

is the FIRST capital letter of the menu text.  Also be

aware that you CANNOT have multiple shortcut keys in a

single menu.

For example

If you have two menu texts that read

TmsRoman 8pt

tmsRoman 10pt

Pressing T will execute the 8 pt;  pressing R would

execute 10 point.

Having the following two entries --

TmsRoman 8pt

TmsRoman 10pt

-- is illegal, and would result in 8 pt *always* being

executed when T was pressed.

The maximum field length is 38 characters.

Editing Printer Control Codes

To the right of each Menu Text is a printer control code.

If you have never cracked the HP manual, best to leave

these alone. If, however, you understand such things,

enjoy.  Just remember that the Escape Code (the little

- 34 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

left pointing arrow visible on this screen) is entered by

holding down CTRL and pressing "[" (Ctrl-LeftBracket).

Also bear in mind that an incorrectly entered control code

will result in what may seem random characters printed on

the first line of the next document you print.  This is

because the Laserjet will understand the correctly entered

portion of a command string, and send the rest of it out

as if it were text and not a command.  So, if you see

something like "l0H" or ")u" in the top left most corner

of a page, go back and check the control code for the menu

item just selected.

The maximum field length is 38 characters.

The ability to customize the menu text and commands of the

Fonts menu is an incredibly useful feature.  For instance,

you could tailor the Fonts selections to encompass all of

your favorite fonts, deleting those you do not have

cartridges for, and adding selections for the cartridges

you do have.  You could also implement downloading of soft

fonts to the Laserjet by entering the appropriate code and

soft font file name.

____________________               3.8.7 Port LPT1/COM1

This setting allows you to tell LSU which port to send

information.

The choices are limited:

LPT1 - parallel port number one

COM1 - serial port number one

The latter choice may be required in situations when you

have two printers, or the Laserjet is connected via a

network accessible only through the serial port.

- 35 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

___________________               3.8.8 Save defaults

One of the nice things about LSU is the ability of the

user to configure several options to suit their needs and

preferences.

This is the command that makes those configuration choices

permanent.  A completely new copy of the program file

LSU.COM is written to disk.

The options that are saved by this command are:

-- Hotkey

-- Force mono

-- Model

-- The Fonts menu text and control codes, including the

Custom/Auto code that is sent to the printer when LSU is

invoked with the "/B" parameter.

-- Port

-- Default Return Address - in order to change the default

return address (the address that is automatically loaded

into the Envelope Editor when you select Envelope.New

envelope) you must first edit the default, then Save

Setup.

NOTE: In order for Laserjet to create a new version of

itself, the program must reside in the currently active

directory and must be named LSU.COM.

If you are running LSU in TSR mode and want to save your

new defaults, but are not currently logged onto LSU's

directory, you must quit the program you are running and

CHDIR to the appropriate directory.

- 36 -

___________________________________________________________          Laserjet Setup Utility 5.04                           Gallo

_________________________           4. Warranties and things.

This program has been tested on an IBM XT and an IBM Model

80.  It should work with most compatible MS-DOS machines.

However, I can make no guarantees about its performance on

your specific hardware/software combination.

I cannot be held liable for any damages, either to

software, hardware, or user files caused directly or

indirectly by the use of Laserjet Setup Utility.

This program is distributed as Shareware.  You are

encouraged to copy and freely distribute Laserjet on the

single condition that the program file be named LSU.COM,

that any archive be named LSUxx.ARC, where xx is the

version number, and that the archive contains this

documentation file.

If after a reasonable trial period, you are still using

LSU, you must obtain a license for continuing to do so.

Registration costs are as follows:

Single, non-commercial license:  25.00

Single, commercial licence:      35.00

Commercial, site licences:

2-10 : 30.00

11-20: 25.00

21 or more: 20.00

Send fee (and appropriate sales tax for New York State

residents) to:

Guy Gallo

219 East 69th Street

New York, NY 10021

- 37 -