Making Novell's NetWare cooperate with other protocols.

To share an Ethernet card with multiple protocol stacks, you must

have a way to direct packets to the appropriate stack.  IPX does not

lend itself to doing this.  Some vendors have tortured their IPX into

giving up packets when it needs to.  Packet drivers (and a packet

driver IPX) are a good solution.  Novell has chosen ODI drivers (and

IPXODI) for NetWare 4.

First, install a packet driver.  Read the other documentation to see

how to do this.  Do not use the -n switch. Then run the PDIPX.COM

found in pdipx103.zip.  Then run your favorite NETx program.

That's it!

Upgrading from BYU packet driver IPX:

The pdipx is not a drop-in replacement for the BYU packet driver IPX.

It does not need the -n switch, because unlike the BYU packet driver

IPX, it understands about different frame types.  By default, it uses

the default frame type -- Novell's "Raw" IEEE 802.3.  If you're using

Ethernet_II, you must set your frame type as explained below.

Updating the IPX:

Pdipx103 has a fairly recent IPX.  If you need to regenerate it with

a newer IPX, then treat the SIPX.LAN and SIPX.OBJ files as

third-party shell drivers.  Perform the SHGEN (NetWare 2) or WSGEN

(NetWare 3) procedure per Novell's documentation.  Set the frame type

as explained below if you're using Ethernet_II framing.

Setting your frame type:

The PDIPX expects Novell's "raw 802.3" packet types by default.  If

you're using Ethernet_II framing, then run econfig on IPX.  Use

"econfig pdipx.com shell:e 8137".  This sets the packet type to

Ethernet and type 8137 (Novell).  Just running "econfig pdipx.com"

will tell you what frame type it's configured for.