GMOUSE.DOC - GENIUS MOUSE Programming Interface

Programmer's Reference Manual

GENIUS-MOUSE DRIVER

Serial Mouse Version

Release 7.01

KUN YING ENTERPRISE CO.,LTD.

RM. 1311,NO.41,SEC 1,

CHUNG HSIAO W. RD.

TAIPEI, TAIWAN, R.O.C.

P.O. BOX: 22826 TAIPEI

TEL: 3813682 3318779 3118545 3822228

TELEX: 11388 KUNYING

FAX: 2-361-5300

page 1

Introduction

The following provides information to programmers who want

to include support for GENIUS-MOUSE in their application

programs. This document includes descriptions of the

graphics and text cursors, and descriptions of the mouse

functions.

This protocol is compatible with the protocol of Microsoft

Corp. as it is documented in the installation and operation

manual published by Microsoft Corp. entitled Microsoft (R)

Mouse User's Guide for IBM (R) Personal Computers.

These notes apply to the GENIUS Serial Mouse Driver

(mouse.com and mouse.sys) Release 5.01.

Cursor Descriptions - Text and Graphics Cursors

The characteristics of the cursor are determined by

Functions 9 and 10. GENIUS MOUSE has both text and graphics

cursors available. The graphics cursor is a shape that

moves over the images on the screen. The shape can include

an arrow, a sight mark, or others. The software text cursor

is a character attribute that moves from character to

character on the screen. The hardware text cursor is a

moving, sometimes flashing block which also moves from

character to character on the screen. The three cursors are

exclusive - only one can be on the screen at one time. You

have the option of choosing which cursor will appear on the

screen and of switching from one cursor to the other.

Graphics Cursor

The graphics cursor is an array of pixels (normally 16 by

16).  It is defined by two arrays of bits called the screen

mask and the cursor mask.

The screen mask is used to determine which of the cursor

pixels becomes are part of the shape (1) or of the

background (0) (it is ANDed with the screen contents).	The

cursor mask is used to determine which of the pixels of the

cursors contribute to the color/shape of the cursor (it is

XORed with the result of the previous operation).

page 2

The operational behaviour of these bit arrays are

summarized by the following table:

screen mask   cursor mask    resulting screen bit

0	       0	      0

0	       1	      1

1	       0	      unchanged

1	       1	      inverted

Note that when a mouse function refers to the graphics

cursor location it is referring to the point on the screen

that lies directly under the cursor's target area. The

target area is the point in the cursor block that the mouse

software uses to determine the cursor coordinates. This

point is called the hot spot of the cursor. The upper left

corner of the cursor block is the hot spot for the default

graphic cursor.

Text Cursor

The text cursor is a cursor that can be used when the video

is in one of the text modes.

Two kind of text cursors are supported: a hardware text

cursor and a software text cursor.

The hardware text cursor is the cursor actually placed on

the screen by the video controller itself. It is defined in

terms of the scan lines of the character cell numbered from

0 starting from the top scan line. The numbers of scan

lines on a character cell depends on the actual video

controller and monitor (see the controller documentation

for details).

The software text cursor is a character or a character

attribute that replaces and/or modifies the character cell

on the screen where it is positioned.

The behaviour of this cursor is defined by 2 16-bit values.

The format of the two values is the following:

bit	 description

15	 blinking (1) or non blinking (0) character

14-12	 background color

11	 high intensity (1) or medium intensity (0)

10-8	 foreground color

7-0	 character code

page 3

The two values are called the screen mask and the cursor

mask.  The screen mask is used to determine which of the

character attributes are preserved (it is ANDed with the

screen character and attribute).  The cursor mask is used

to determine which of the characteristics are changed by

the cursor (it is XORed with the result of the previous

operation).

Notice to System Programmers

System programmers who intercept the video i/o software

interrupt (int 10h) should exercise utmost care, and note

the following:

The program should guarantee that video mode changes

(function 0) are intercepted by the mouse driver. The mouse

driver guarantees consistency of the screen contents to

programs that write on the screen via int 10h (or via

DOS).

Some programs write directly on the video memory.  In this

case the application program should use function 2 (hide

cursor) or 16 (conditional hide cursor) before writing,

then function 1 (show cursor) after writing.  GWBASIC is an

example of such a program.

The presence of the mouse driver can also be detected by

checking the interrupt vector entry for interrupt 51 in low

memory for value 0 or for pointing to an IRET instruction.

This method should be used with some care as it is

dependent on the release of DOS.

The programming interface uses the 8086 software interrupt

51. Parameter values are passed and returned via 8086

registers. This interface can be used from assembly

language programs or from high level language programs such

as BASIC , FORTRAN , COBOL , C or Pascal.

You can also let the mouse software call a subroutine in your

program whenever a specific condition occurs. When this

capability is enabled, the mouse software interrupts whatever

process is going on and passes execution control to the

subroutine that you have specified in Function 12 of the mouse

system calls.

page 4

Making Calls From Assembly Language Programs :

TO make mouse systemcalls from an assembly language program,

you must:

1. Load the AX, BX, CX and DX registers with the parameter

values.

2. Execute software interrupt 51(33H)

The AX, BX, CX and DX registers correspond to the M1%,

M2%, M3%, and M4% parameters. defined for the BASIC program.

Values returned by the Mouse functions will be placed in the

registers.

Example

Use the following instructions to set the cursor position to 150

(horizontal) and 100 (vertical):

; * set cursor to location (150,100)

Mov  AX,4	   ;(function call 4)

Mov  CX,150   ;(set Horiziontal to 150)

Mov  DX,100   ;(set vertical to 100)

Int  51	   ;(interrupt to mouse)

Note: When making a mouse system call in Assembly Language,

function 9 and 12 expect a somewhat different.

Value for the fourth parameter than when calling from a

Basic program.

page 5

Making Calls from BASIC (BASICA) Language Program :

It is possible to make a mouse system call from a BASIC

program running under the GWBASIC interpreter.	The

following shows how to make these calls.

Insert an initialization sequence such as:

10	DEF SEG=0

20	MSEG=256*PEEK(51*4+3)+PEEK(51*4+2)

30	MOUSE=256*PEEK(51*4+1)+PEEK(51*4)+2

40	DEF SEG=MSEG

Be sure that the statements appear before any calls to

mouse functions.  Then use the CALL statement to make the

call.

CALL MOUSE(M1%,M2%,M3%,M4%)

where MOUSE is the variable containing the entry offset of

the mouse software, and M1%, M2%, M3%, and M4% are the

names of the integer variables you have chosen for

parameters in this call.  They correspond to the values for

AX, BX, CX, and DX which are described in this document.

As an example:

100  'Set minimum and maximum horizontal position to

'(320,100)

200  M1%=7 'function number is 7

300  M3%=0 'minimum coordinate

400  M4%=639 'maximum coordinate

500  CALL MOUSE(M1%,M2%,M3%,M4%)

page 6

List of Mouse Functions

The following functions apply to GENIUS MOUSE. These functions

will be described in greater detail in subsequent sections.

FUNCTIONS		FUNCTION NUMBER

Base Functions

Mouse Initialization				0

Show Cursor					1

Hide Cursor					2

Get Mouse Position & Button Status		3

Set Mouse Cursor Position			4

Get Button Press Information			5

Get Button Release Information			6

Set Minimum & Maximum X Position		7

Set Minimum & Maximum Y Position		8

Define Graphics Cursor Block			9

Define Text Cursor			       10

Read Mouse Motion Counters		       11

Define Event Handler			       12

Light Pen Emulation Mode On		       13

Light Pen Emulation Mode Off		       14

Set Mouse Motion/Pixel Ratio		       15

Condition Off				       16

Set Double-Speed Threshold		       19

page 7							    page 6

Description of the Mouse Functions

The following descriptions of the mouse functions specify

the required input and the expected output of each

function. The descriptions also include any special

circumstances involved with each function. The input and

output parameters are specified as values for 8086

registers and parameter for BASIC Language.

Mouse Initialization - Function 0

***********************************

This function describes whether the mouse hardware and

software are installed. The mouse status is 0 if the mouse

hardware and software are not installed and -1 if the

hardware and software are installed.

8086 register :

Input

AX = 0

Output

AX = mouse status

BX = number of buttons

Basic:

Input

M1% = 0

Output

M1% = Mouse status

M2% = number of Buttons

Example:

100  :  Is mouse present ? if not error

200     M1% = 0

300     CALL Mouse(M1%,M2%,M3%,M4%)

500     IF NOT (M1%) THEN PRINT "Mouse not installed" : END

page 8

Show Cursor - Function 1

**************************

This function increments the internal cursor flag. If the

flag is 0 it displays the cursor on the screen. The cursor

tracks the motion of the mouse, changing position as the

mouse changes position.

8086 register :

Input

AX = 1

Output

none

Basic:

Input

M1% = 1

Output

none

Example:

100  :  show the cursor

200     M1% = 1

300     CALL Mouse(M1%,M2%,M3%,M4%)

page 9

Hide Cursor - Function 2

**************************

This function removes the cursor from the screen and

decrements the internal cursor flag. Although the cursor is

hidden it still tracks the motion of the mouse, changing

position as the mouse changes position.

8086 register :

Input

AX = 2

Output

none

Basic:

Input

M1% = 2

Output

none

Example:

100  :  Hide the cursor

200     M1% = 2

300     CALL Mouse(M1%,M2%,M3%,M4%)

page 10

Get Mouse Position & Button Status - Function 3

*************************************************

This function reports the status of the buttons. It also

reports the position of the cursor horizontally and

vertically. The button status is a single integer value

with bit 0 representing the left button and bit 1 the

right. When the button is down a bit is 1 and when the

button is up a bit is 0.

8086 register :

Input

AX = 3

Output

BX = button status

CX = horizontal cursor position

DX = vertical cursor position

Basic :

Input

M1% = 3

Output

M2% = button status

M3% = cursor position

M4% = cursor position

Example :

100  :  Get current cursor positions, check button status

200     M1% = 3

300     CALL Mouse(M1%,M2%,M3%,M4%)

400     IF M2% AND 1 THEN PRINT "LEFT Button Down"

500     IF M2% AND 2 THEN PRINT "RIGHT Button Down"

page 11

Set Mouse Cursor Position - Function 4

****************************************

This function sets the cursor to the specified horizontal

and vertical positions on the screen. The new values must

be within the specified ranges of the virtual screen. If

you are not using a high resolution screen the values are

rounded to the nearest values permitted by the screen for

horizontal and vertical positions.

8086 register :

Input

AX = 4

CX = new horizontal cursor position

DX = new vertical cursor position

Output

none

Basic:

Input

M1% = 4

M3% = (horixontal) new cursor position

M4% = (vertical) new cursor position

Example:

100  ;  Put corsor in center of screen

200     M1% = 4

300     M3% = INT(Hmax/2)

400     M4% = INT(Vmax/2)

500     CALL Mouse(M1%,M2%,M3%,M4%)

page 12

Get Button Press Information - Function 5

*********************************************

This function reports on the current button status.  It

gives a count of button presses since the last call to

Function 5, and it gives the horizontal and vertical

position of the cursor the last time the button was

pressed.

The BX parameter indicates which button was pressed. If BX

is 0, the left button is checked.  If BX is 1, the right

button is checked. The button status is a single integer

value. Bit 0 represents the left button and bit 1 the

right. If the button is down a bit is 1 and if it is up a

bit is 0.

8086 register :

Input

AX = 5

BX = button

Output

AX = button status

BX = number of button presses

CX = horizontal cursor position at last press

DX = vertical cursor position at last press

Basic :

Input

M1% = 5

M2% = Button

Output

M1% = Button status

M2% = number of Button presses

M3% = horizontal cursor position at last press

M4% = vertical cursor positin at last press

Example :

100  ;  Get cursor position at last button press

200     M1% = 5

300     M2% = 0

400     CALL Mouse(M1%,M2%,M3%,M4%)

500     IF (M1% AND 1) THEN PRINT "LEFT Button Down"

page 13

Get Button Release Information - Function 6

*********************************************

This function reports on the current button status. It

reports a count of the button releases since the last call

to Function 6, and it gives the horizontal and vertical

position of the cursor since the last time the button was

released.

The BX parameter specifies which button is checked. If BX

is 0, the left button is checked. If BX is 1, the right

button is checked. The button status is a single integer

value. Bit 0 represents the left button and bit 1

represents the right button. If a button is down a bit is 1

and if a bit is 0 a button is up.

8086 register :

Input

AX = 6

BX = button

Output

AX = button status

BX = number of button releases

CX = horizontal cursor position at last release

DX = vertical cursor position at last release

Basic :

Input

M1% = 6

M2% = Button

Output

M1% = button status

M2% = number of button releases

M3% = horizontal cursor position at last release

M4% = vartical cursor position at last release

Example :

100  ; Get cursor position at last button release

200    M1% = 6

300    M2% = 1

400    CALL Mouse(M1%,M2%,M3%,M4%)

500    IF (M1% AND 2) THEN PRINT "RIGHT Button Down"

page 14

Set Minimum & Maximum X Position - Function 7

************************************************

This function sets the minimum and maximum possible cursor

positions on the screen. All cursor movement is restricted

to this area once the area is set. The virtual screen

determines the maximum and minimum values.

If the cursor is outside the defined area when the call is

made, it moves to just inside the area. If the minimum

value is greater than the maximum the two values are

exchanged.

8086 register :

Input

AX = 7

CX = new minimum horizontal cursor position

DX = new maximum horizontal cursor position

Output

none

Basic :

Input

M1% = 7

M3% = new minimum horizontal cursor position

M4% = new maximum horizontal cursor position

Output

none

Example :

100  ;  Limit cursor to horizontal position between

50 and 200

200     M1% = 7

300     M3% = 50

400     M4% = 200

500     CALL Mouse(M1%,M2%,M3%,M4%)

page 15

Set Minimum & Maximum Y Position - Function 8

***********************************************

This function sets the minimum and maximum possible cursor

positions on the screen. All cursor movement is restricted

to this area once the area is set. The virtual screen

determines the maximum and minimum values.

If the cursor is outside the defined area when the call is

made, it moves to just inside the area. If the minimum

value is greater than the maximum the two values are

switched.

8086 register :

Input

AX = 7

CX = new minimum vertical cursor position

DX = new maximum vertical cursor position

Output

none

Basic :

Input

M1% = 7

M3% = new minimum vertical cursor position

M4% = new Maximum vertical cursor position

Output

none

Example :

100  ;  Limit cursor to vertical position between

100 and 200

200     M1% = 8

300     M3% = 100

400     M4% = 200

500     CALL Mouse(M1%,M2%,M3%,M4%)

page 16

Define Graphics Cursor Block - Function 9

*******************************************

This function determines what the shape and color of the

cursor will be when it is in graphics mode, and it

identifies the center of the cursor. The cursor block is a

16 X 16 bit pattern. The cursor hot spot values must be

within the range of -16 to 16. They define one pixel within

(or outside) the cursor block.

8086 register :

Input

AX = 9

BX = horizontal cursor hot spot

CX = vertical cursor hot spot

DX = pointer toscreen and cursor mask

Output

none

Basic :

Input

M1% = 9

M2% = horizontal cursor hot spot

M3% = vertical cursor hot spot

M4% = pointer to sereen and cursor mask

page 17

Example :

100  ;

200  ;  Define the screen mask

300  ;

400     cursor(0,0)  = &hffff	  '1111111111111111

500     cursor(1,0)  = &hffff	  '1111111111111111

600     cursor(2,0)  = &hffff	  '1111111111111111

700     cursor(3,0)  = &hffff	  '1111111111111111

800     cursor(4,0)  = &hffff	  '1111111111111111

900     cursor(5,0)  = &hffff	  '1111111111111111

1000     cursor(6,0)  = &hffff	  '1111111111111111

1100     cursor(7,0)  = &hffff	  '1111111111111111

1200     cursor(8,0)  = &hffff	  '1111111111111111

1300     cursor(9,0)  = &hffff	  '1111111111111111

1400     cursor(10,0) = &hffff	  '1111111111111111

1500     cursor(11,0) = &hffff	  '1111111111111111

1600     cursor(12,0) = &hffff	  '1111111111111111

1700     cursor(13,0) = &hffff	  '1111111111111111

1800     cursor(14,0) = &hffff	  '1111111111111111

1900     cursor(15,0) = &hffff	  '1111111111111111

2000  ;

2100  ;  Define the cursor mask

2200  ;

2300     cursor(0,1)  = &h8000	   '1000000000000000

2400     cursor(1,1)  = &hf000	   '1110000000000000

2500     cursor(2,1)  = &hf800	   '1111100000000000

2600     cursor(3,1)  = &hff00	   '1111111000000000

2700     cursor(4,1)  = &hd800	   '1101100000000000

2800     cursor(5,1)  = &h0c00	   '0000110000000000

2900     cursor(6,1)  = &h0600	   '0000011000000000

3000     cursor(7,1)  = &h0300	   '0000001100000000

3100     cursor(8,1)  = &h0000	   '0000000000000000

3200     cursor(9,1)  = &h0000	   '0000000000000000

3300     cursor(10,1) = &h0000	   '0000000000000000

3400     cursor(11,1) = &h0000	   '0000000000000000

3500     cursor(12,1) = &h0000	   '0000000000000000

3600     cursor(13,1) = &h0000	   '0000000000000000

3700     cursor(14,1) = &h0000	   '0000000000000000

3800     cursor(15,1) = &h0000	   '0000000000000000

3900  ;

4000  ;  Set the mouse cursor shape, color, and

4050  ;  hot spot

4100  ;

4200     M1% = 9

4300     M2% = 0 'horixontal hot spot

4400     M3% = 0 'vartical hot sopt

4500     CALL Mouse(M1%,M2%,M3%,cursor(0,0))

page 18

Define Text Cursor - Function 10

**********************************

This function selects the software or hardware cursor. When

the hardware cursor is selected this function sets the

first and last scan lines which will be shown on the

screen. The value of the parameter BX selects the cursor

type. If the value is 0, the software text cursor is

selected. If the value is 1, the hardware text cursor is

selected. When the software text cursor is selected, the

values of parameters CX and DX specify the screen and

cursor masks. When the hardware cursor is selected, the

parameters of CX and DX must contain the line numbers of

the first and last scan line in the cursor to be shown on

the screen.

8086 register :

Input

AX = 10

BX = select cursor

CX = screen mask value/scan line start

DX = cursor mask value/scan line stop

Output

none

Basic :

Input

M1% = 10

M2% = select cursor

M3% = screen mask value/scan line start

M4% = cursor mask value/scan line stop

Example :

TO create a text crusor that inverts the foregrand

and backgrand colors:

100     M1% = 10

110     M2% = 0

120     M3% = &hffff

130     M4% = &h7700

140     CALL Mouse(M1%,M2%,M3%,M4%)

page 19

Read Mouse Motion Counters - Function 11

******************************************

This function returns the horizontal and vertical step

count, which is the distance the mouse has moved in 1/200

inch increments, since the last call to this function.

With the Default Accelerator installed the ratio of mouse

to cursor movement is variable between 1/200 and 1/400.

The step count is always within the range -32768 to 32767.

A positive horizontal count specifies a motion to the right

while a positive vertical count specifies a motion to the

bottom of the screen. The step count is set to 0 after the

call is completed.

8086 register :

Input

AX = 11

Output

CX = horizontal count

DX = vertical count

Basic :

Input

M1% = 11

Output

M3% = horizontal count

M4% = vertical count

Example :

100  ;  Get the Michey count

200     M1% = 11

300     CALL Mouse(M1%,M2%,M3%,M4%)

page 20

Define Event Handler - Function 12

************************************

This routine sets the call mask and subroutine address for

the mouse software interrupts. The software interrupts stop

execution of your program and call the specified subroutine

whenever one or more of the conditions defined by the call

mask occur. The call mask is a single integer value which

defines the conditions which will cause an interrupt. Each

bit in the call mask corresponds to a specific condition:

Mask Bit  Condition

0	      change cursor position

1	      press left button

2	      release left button

3	      press right button

4	      release right button

5-15      not used

8086 register :

Input

AX = 12

CX = call mask

ES:DX = address of subroutine

Output

none

Basic :

Input

M1% = 12

M3% = call mask

M4% = ADDRESS offset to subroutine

Output

none

Example :

100  ;   Call subroutine SKETCH on Left Button press

200      M1% = 12

300      M3% = &h0002

400      M4% = SKETCH%

500      CALL Mouse(M1%,M2%,M3%,M4%)

page 21

Light Pen Emulation Mode On - Function 13

********************************************

This function enables light pen emulation by the mouse.

When the mouse emulates the light pen, calls to the pen

function will return the cursor position at the last pen

down which is controlled by the mouse buttons. The pen is

down when both buttons are down. The pen is off the screen

when both buttons are up.

8086 register :

Input

AX = 13

Output

none

Basic :

Input

M1% = 13

Output

none

Example :

100  ;  Enable Light pen

200     M1% = 13

300     CALL Mouse(M1%,M2%,M3%,M4%)

page 22

Light Pen Emulation Mode Off - Function 14

*******************************************

This function disables the light pen emulation mode. When

light pen simulation is disabled, calls to the pen function

return information about the light pen only.

8086 register :

Input

AX = 14

CX = new horizontal cursor position

DX = new vertical cursor position

Output

none

Basic :

Input

M1% = 14

M3% = new horizontal cursor position

M4% = new vertical cursor position

Example :

100  ;  Disable Light pen

200     M1% = 14

300     CALL Mouse(M1%,M2%,M3%,M4%)

page 23

Set Mouse Motion/Pixel Ratio - Function 15

*********************************************

This function sets the mouse motion to screen pixel ratio.

The horizontal and vertical ratios specify the amount of

mouse motion at 8 pixels with the values falling within the

range of 1 to 32767. The default values are 8 steps to 8

pixels horizontally and 16 steps to 8 pixels vertically.

This is equivalent to 3.2 inches of horizontal mouse

movement and 2.0 inches of vertical mouse movement.

8086 register :

Input

AX = 15

CX = horizontal step to pixel ratio

DX = vertical step to pixel ratio

Output

none

Basic :

Input

M1% = 15

M3% = horizontal step to pixel ratio

m4% = vertical step to pixel ratio

Output

none

Example :

100  ;  Set mickey/pixel ratio at 16 to 8 AND 32 TO 8

200     M1% = 15

300     M3% = 16

400     M4% = 32

500     CALL Mouse(M1%,M2%,M3%,M4%)

page 24

Conditional Off - Function 16

*******************************

This function allows the user to define an area on the

screen within which the mouse will turn off. Performing the

Show Cursor Function - Function 1 - resets the region and

turns the mouse back on. This function is used to guard a

portion of the screen which your program is about to

update.

8086 register :

Input

AX = 16

CX = left margin

DX = top margin

SI = right margin

DI = bottom margin

Output

none

BASIC :

Input

M1% = 16

M4% = address of the region array

Output

none

Example :

100 ; define screen region for conditional off

200  OFF%(1)=10

300  OFF%(2)=30

400  OFF%(3)=40

500  OFF%(4)=80

600  M1%=16

700  CALL MOUSE(M1%,M2%,M3%,off(0))

800 ; Screen update routine

:

:

2000  M1%=1

2100  CALL MOUSE (M1%,M2%,M3%,M4%)

page 25

Set Double-Speed Threshold - Function 19

*****************************************

Function 19 sets the threshold speed for doubling the

cursor's motion on the screen. Using function 19 makes it

easier to point at images widely separated on the screen.

Parameter M4% defines the mouse's threshold speed. If

no value is given, or if the mouse is reset by a call to

function 0, a default value of 64 mickeys per second is

assigned. If the mouse moves faster than the value of M4%,

cursor motion doubles in speed. The threshold speed remains

set until function 19 is called again or until the mouse is

reset by function 0.

Once your program turns on teh speed-doubling feature, this

feature is always on. However, the program can effectively

turn of this feature by setting M4% to a speed faster than

the mouse can physically move (for example, 10,000) and then

calling function 19.

8086 register :

Input

AX = 19

DX = threshold speed in mickeys/second

Output

none

BASIC :

Input

M1% = 19

M4% = threshold speed in mickeys/second

Output

none

page 26

Example :

100	;

110	;  Set threshold to 32 mickeys/sec

120	;

130	   M1% = 19

140	   M4% = 32

150	   CALL Mouse (M1%,M2%,M3%,M4%)

.

.

.

.

1000 ;  Turn off speed doubling

1010    M1% = 19

1020    M4% = 1000  'mickeys/second

1030    CALL Mouse(M1%,M2%,M3%,M4%)

***************  END  ***************