1-2-3 NOTES                                      All Versions

Note 101                                        June 22, 1984

AUTOMATIC POINTER MOVEMENT MACROS

:::::::::::::::::::::::::::::::::

SUMMARY

An  automatic pointer movement macro automatically moves the cell

pointer each time you press the Return key.  It is helpful if you

need to make a number of entries in a column or row.

This  macro  enables  you to move the  pointer  while  using  the

numeric  keypad  to  make entries.  It saves you the  trouble  of

having to turn the numeric keypad on and off to move the pointer.

(Pressing the Num Lock key so that you can use the numbers on the

numeric keypad disables the pointer movement, or arrow, keys.)

This  macro is also helpful if you have a long list of labels  to

enter in a column or row. It does not save keystrokes per se, but

it  does allow you to type the list without moving your hand  off

the standard typewriter keys to press an arrow key.

Once an automatic pointer movement macro is written, it is simple

to use:  activate the macro, type your data, and press the Return

key.

This note describes four macros, one for each direction:

* the Up Macro             * the Left Macro

* the Down Macro           * the Right Macro

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::

MACRO COMMANDS

An  automatic  pointer movement macro has  the  following  format

(this is the Down Macro):

{?}{down}/xg\D~

The following macro commands are used:

{?}

This  symbol  tells the macro program to pause until you type  in

the necessary data.  After you type the data and press the Return

key,  the  program reads the macro instructions that  follow  the

{?}.

{up}{down}{left}{right}

The  pointer  movement keys (or arrow keys) are specified in  the

macro  by  name.  The macro moves the pointer  in  the  direction

indicated.

1

/xgLOCATION~

This  command  sends the macro program to another cell  or  range

location  where  it  will continue to read  keystrokes.  The  /xg

command is used to create the loop necessary to move the  pointer

elsewhere  in  the  same column or row;  it  actually  sends  the

program  back  to the beginning of the macro,  enabling the  same

macro instructions to be used repeatedly.

\

A backslash followed by a single letter is a "macro range  name."

Use  the  Range Name Create command to name macro with the \  and

any letter of the alphabet.

~

The tilde stands for the Return key in a macro. It must be placed

at any point in the macro sequence where you would normally press

the Return key if you were entering data and commands manually.

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::

The Down Macro

--------------

This  macro moves the pointer down one cell after you  press  the

Return key.

1.  Type the macro in one cell:

{?}{down}/xg\D~ (press RETURN)

2.  Give  the macro the range name \D with the /Range Name Create

Command.  Place  the  pointer on the cell that  contains  the

macro and type:

/rnc\D (press RETURN 2 times)

The Up Macro

------------

This  macro  moves  the pointer up one cell after you  press  the

Return key.

1.  Type the macro in one cell:

{?}{up}/xg\U~ (press RETURN)

2.  Name  the  macro  \U.  Place the pointer  on  the  cell  that

contains the macro and type:

/rnc\U (press RETURN 2 times)

2

The Right Macro

---------------

This  macro  moves  the pointer one cell to the right  after  you

press the Return key.

1.  Type the macro in one cell:

{?}{right}/xg\R~ (press RETURN)

2.  Name  the  macro  \R.  Place the pointer  on  the  cell  that

contains the macro and type:

/rnc\R (press RETURN 2 times)

The Left Macro

--------------

This macro moves the pointer one cell to the left after you press

the Return key.

1.  Type the macro in one cell:

{?}{left}/xg\L~ (press RETURN)

2.  Name  the  macro  \L.  Place  the pointer on  the  cell  that

contains the macro and type:

/rnc\L (press RETURN 2 times)

Note:    The macro is entered as a label.  As you type the  macro

it  appears on the status line (in the upper left corner

of the monitor).  Notice that 1-2-3 automatically enters

the apostrophe (') that denotes a label before the macro

itself. You don't have to type the ' character.

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::

USING THE MACROS

To use one of the macros:

1.   Position the pointer in the cell in which you want to  enter

data.

2.   Simultaneously  press the Alt key and one macro letter name:

D,  U,  R, L. (You do not have to type a capital letter when

using the macro).

3.   Type the data and press the Return key.

3

4.   To get out of the macro and return to the 1-2-3 Ready  mode,

simultaneously  press the Ctrl and Break (Scroll Lock) keys.

An  automatic pointer movement macro contains  a  continuous

loop.  Thus, this procedure is necessary to break out of the

loop and end the macro (i.e., turn it off).

To use a macro with the numeric keypad, simply press the Num Lock

key  and then do the same as above.  The pointer will move in the

specified  direction,  and the number keys can be used  to  enter

values in the worksheet.

4

###############################################################################################################################