AS31(1L)            UNIX Programmer's Manual             AS31(1L)

NAME

as31 - An Intel 8031/8051 assembler

SYNOPSIS

as31 [ -Fformat ] [ -Aarg ] [ -l ] infile.asm

DESCRIPTION

_#A_#s_#3_#1 assembles _#i_#n_#f_#i_#l_#e._#a_#s_#m into one of several different out-

put formats. The output will be in a file called infile.obj.

The .asm extenstion is required.

OPTIONS

The options must appear before the input file name. Both

options are optional. The text of each flag must appear on

the same argument as the flag. For example, "-Fod" is a

valid argument, but "-F od" is not.

-_#F_#f_#o_#r_#m_#a_#t

This options specifies the output format that is to be

used.

Currently the only options available for this are:

tdr  This format generates an asci file of hex digits

formatted in such a way, so that they can be read

by tdr's debugger. An argument can be specified

(See -A option) which will pass a format specific

string to the format generator. In this case, the

argument string represents an offset to add to the

location counter. This offset is specified in

decimal and defaults to 64*1024 (0x10000). To

specify and offset of 100, you would need "-Ftdr

-A100" when invoking the assembler.

byte This format is simply an address and a byte on

each line, in ascii.  No -A option is used.

od   This format is similar to the output from od(1).

The format consists of an address followed by six-

teen hexadecimal bytes, followed by the equivilant

ASCII. No -A option is used.

srec2, srec3, srec4

The srecord generator is capable of generating

output with any one of 2, 3, or 4 byte addresses.

The -A option can be used to set the base address

offset, the default here is 0x0000 (unlike tdr).

Printed 11/30/91                                                1

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

NOTE: This assembler allows for the output formats to

be expanded to include many different output formats.

-Aarg

This option specifies a format specific string which is

passed to the format generator. Both format "tdr" and

the srecord formats use this option.

-l   This option tells the assembler to also generate a

listing file.  A listing will be placed in the file

infile.lst. Where 'infile' is the file that is being

assembled. This option may appear anywhere before

infile.asm. The option must occur isolated on the com-

mand line.

The listing file shows the assembler generated code in

hex, and up to 60 characters are retained from the

source file.

#9

ASSEMBLER INSTRUCTIONS

This assembler accepts standard 8031/8051 instruction for-

mats.  Below is a list of instructions and addressing modes.

INSTRUCTION                             BYTES     CYCLES

-----------                             -----     ------

ACALL     addr11                        2         24

ADD       A, #data8                     2         12

ADD       A, @Ri                        1         12

ADD       A, Rn                         1         12

ADD       A, direct                     2         12

ADDC      A, #data8                     2         12

ADDC      A, @Ri                        1         12

ADDC      A, Rn                         1         12

ADDC      A, direct                     2         12

AJMP      addr11                        2         24

ANL       A, #data8                     2         12

ANL       A, @Ri                        1         12

ANL       A, Rn                         1         12

ANL       A, direct                     2         12

ANL       C, /bit                       2         24

ANL       C, !bit                       2         24

ANL       C, bit                        2         24

ANL       direct, #data8                3         24

ANL       direct, A                     2         12

CJNE      @Ri, #data8, rel              3         24

CJNE      A, #data8, rel                3         24

CJNE      A, direct, rel                3         24

CJNE      Rn, #data8, rel               3         24

CLR       A                             1         12

CLR       C                             1         12

Printed 11/30/91                                                2

#9

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

CLR       bit                           2         12

CPL       A                             1         12

CPL       C                             1         12

CPL       bit                           2         12

DA        A                             1         12

DEC       @Ri                           1         12

DEC       A                             1         12

DEC       DPTR                          1         12

DEC       Rn                            1         12

DEC       direct                        2         12

DIV       AB                            1         48

DJNZ      Rn, rel                       2         24

DJNZ      direct, rel                   3         24

INC       @Ri                           1         12

INC       A                             1         12

INC       DPTR                          1         24

INC       Rn                            1         12

INC       direct                        2         12

JB        bit, rel                      3         24

JBC       bit, rel                      3         24

JC        relative                      2         24

JMP       @A + DPTR                     1         24

JMP       @DPTR + A                     1         24

JNB       bit, rel                      3         24

JNC       relative                      2         24

JNZ       relative                      2         24

JZ        relative                      2         24

LCALL     addr16                        3         24

LJMP      addr16                        3         24

MOV       @Ri, #data8                   2         12

MOV       @Ri, A                        1         12

MOV       @Ri, direct                   2         24

MOV       A, #data8                     2         12

MOV       A, @Ri                        1         12

MOV       A, Rn                         1         12

MOV       A, direct                     2         12

MOV       C, bit                        2         12

MOV       DPTR, #data16                 3         24

MOV       Rn, #data8                    2         12

MOV       Rn, A                         1         12

MOV       Rn, direct                    2         24

MOV       bit, C                        2         24

MOV       direct, #data8                3         24

MOV       direct, @Ri                   2         24

MOV       direct, A                     2         12

MOV       direct, Rn                    2         24

MOV       direct, direct                3         24

MOVC      A, @A + DPTR                  1         24

MOVC      A, @A + PC                    1         24

MOVC      A, @DPTR + A                  1         24

MOVC      A, @PC + A                    1         24

MOVX      @DPTR, A                      1         12

Printed 11/30/91                                                3

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

MOVX      @Ri, A                        1         24

MOVX      A, @DPTR                      1         24

MOVX      A, @Ri                        1         24

MUL       AB                            1         48

NOP                                     1         12

ORL       A, #data8                     2         12

ORL       A, @Ri                        1         12

ORL       A, Rn                         1         12

ORL       A, direct                     2         12

ORL       C, /bit                       2         24

ORL       C, !bit                       2         24

ORL       C, bit                        2         24

ORL       direct, #data8                3         24

ORL       direct, A                     2         12

POP       direct                        2         24

PUSH      direct                        2         24

RET                                     1         24

RETI                                    1         24

RL        A                             1         12

RLC       A                             1         12

RR        A                             1         12

RRC       A                             1         12

SETB      A                             1         12

SETB      bit                           2         12

SJMP      relative                      2         24

SUBB      A, #data8                     2         12

SUBB      A, @Ri                        1         12

SUBB      A, Rn                         1         12

SUBB      A, direct                     2         12

SWAP      A                             1         12

XCH       A, #data8                     2         12

XCH       A, @Ri                        1         12

XCH       A, Rn                         1         12

XCH       A, direct                     2         12

XCHD      A, #data8                     2         12

XCHD      A, @Ri                        1         12

XCHD      A, Rn                         1         12

XCHD      A, direct                     2         12

XRL       A, #data8                     2         12

XRL       A, @Ri                        1         12

XRL       A, Rn                         1         12

XRL       A, direct                     2         12

XRL       direct, #data8                3         12

XRL       direct, A                     2         12

ASSEMBLER DIRECTIVES

As31 includes the following assembler directives:

.ORG expr

Start assembling at the address specified by the

expression expr.  An error occurs if the assembler

Printed 11/30/91                                                4

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

starts assembling over an address space that has previ-

ously been assembled into.

.EQU symbol, expr

Set symbol to the value of expr. The value for expr

must be known during the first pass, when the line con-

taining the .EQU is encountered.

.BYTE expr, expr, ...

Assemble the bytes specified by the expression into

memory. A string may also be specified with this direc-

tive.

.WORD expr, expr, ...

Assemble the words specified by the expression into

memory.  The byte ordering used, is that used by the

8031.

.FLAG symbol1, symbol.[0-7]

Sets symbol1 to the bit address specified by the

symbol.[0-7] expression. Where [0-7] denotes a charac-

ter between 0 and 7.  The resulting bit address is

checked to see if it is a valid bit address.

.END This directive is ignored.

.SKIP expr

Adds the value of expr to the location counter. Used to

reserve a block of uninitialized data. Expr should be

in bytes.

LEXICAL CONVENTIONS

-    All characters following a semi-colon are ignored until

a newline is encountered.

-    All numbers default to decimal, unless the number

starts with one of the following:

0x or 0X

This indicates a hexadecimal number. ie. 0x00ff

0b or 0B

This indicates a binary number. (1's and 0's). ie.

0b1100110010

Printed 11/30/91                                                5

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

0    This indicates an octal number. ie. 0377

-    All numbers default to decimal, unless the number ends

with one of the following characters:

b or B

This indicates a binary number. Unless 0x was used

above.  ie. 1010101b

h or H

This always indicates a hex number, However the if

the first character is non-numerical, then either

0x or 0 must be specified.  This avoids confusing

the assembler into thinking a hex number is a sym-

bol.  For example: 0ffh, 0xffh, 0XffH, 20h, 0x20

and 020h are means to specify a valid hexdigit.

But the following are not: ffh, 0ff.

d or D

This forces a number to decimal. Unless 0X was

used. ie. 129d

o or O

This causes the number to be interpreted as octal.

ie. 377o

-    A character constant can be entered as 'c' where c is

some character. \b, \n, \r, \t, \' \0 are also valid. A

character constant can be used anywhere that an integer

value can.

-    A string is entered as a set of characters enclosed in

double quotes "".  A string is only valid with the

.BYTE directive. \b, \n, \r, \t, \" are also valid

escapes. However \0 is not.

-    Instructions, directives, and the symbols: R0, R1, R2,

R3, R4, R5, R6, R7, A, AB, and C can be entered in

upper or lower case without assembler confusion. These

words however cannot be defined as a user symbol.  Any

user symbol may be used, and case will be preserved. So

the user symbols "foo" and "Foo" are different, but

"addc" is the same as "aDdC".

-    A symbol can be any alpha numerical character plus the

underscore ('_').

Printed 11/30/91                                                6

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

-    Expressions are accepted in most places where a value

or a symbol is needed. An expression consists of the

following operators. All operators evaulate to integer

objects (higher precedence opertors listed first):

-    Unary minus

&    Bit-wise AND.

|    Bit-Wise OR.

*    Integer multiplication.

\ Integer division

%    Intieger modulus

+    Integer addition.

-    Integer subtraction.

-    In addition to these operators, a special symbol '*'

may be used to represent the current location counter.

EXAMPLES

Below is a sample assembly program.

.org    0

start:          mov     P3, #0xff       ; use alternate fns on P3

; leds on P1 are inverted.

setb    F0              ; climbing up

mov     A, #0x01        ; initial bit

write:          cpl     A               ; write it

mov     P1, A

cpl     A

acall   delay

jb      F0, climbup     ; climbing which way?

climbdn:        rr      A               ; down - shift right

jnb     ACC.0, write    ; back for more

setb    F0

ajmp    write

climbup:        rl      A               ; up - shift left

jnb     ACC.7, write    ; back for more

clr     F0

ajmp    write

.end                              ; this directive ignored.

Printed 11/30/91                                                7

AS31(1L)            UNIX Programmer's Manual             AS31(1L)

AUTHOR

Ken Stauffer (University of Calgary)

stauffer@cpsc.ucalgary.ca

Printed 11/30/91                                                8

###########